FORMATIVE 1

Due to escalating conflict between the US and China, your unit has been tasked with deploying personnel, vehicles, and palletized equipment into Ninoy Aquino Intl, a strategic location within the INDOPACOM theater. Using the following aircraft, C-130H and C-130J, you will create a load plan and ensure the below personnel/equipment have been loaded IAW DTR, ATTLA, and Aircraft limitations.

Aircrafts (STD-AL)

- C-130H x2/ 25,000

- C-130J / 40,000

Mission Type: Mobility

Chapter 3: Yes (75% Fuel)

Unit Airlift: TF DELTA

Departure Airfield: Yokota AB (RJTY)

Destination: Ninoy Aquino Intl (RPLL)

Passengers: 26 at 210 lbs. each (2 minimum each chalk)

Passenger Bags: 190 LBS per passenger

-----------------------------------------------------------------------------------------

Your unit UMO has given you a compiled list of items that have been identified as needing to be deployed with the unit. Create 1 cargo list with the following items.

| Description | LIN # | Lin Index # | Weight |
| --- | --- | --- | --- |
| Pallet, 463L x3 | YA0038 | 01 | 4500 LB |
| M1102 | T95924 | 02 | 1400 LBS |
| M1079A1 | T93484 | 08 | 18834 LBS |
| M142 | H53326 | 02 | 28560 LBS |
| M1165A1 | T56383 | 03 | 9870 LBS |

Your unit has weighed and marked the equipment in your unit’s inventory that are identified to deploy.  Update the items in your cargo list according to the below dimensions and weights.

| Description | LIN # | Lin Index # | Weight | Length | Width | Height | Distance (IN) | Axles (lbs) | HAZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pallet, 463L | YA0038 | 01 | 3250 | 108 | 88 | 76 | - | - | - |
| Pallet, 463L | YA0038 | 01 | 2800 | 108 | 88 | 76 | - | - | - |
| Pallet, 463L | YA0038 | 01 | 7000 | 108 | 88 | 96 | - | - | - |
| M1102 | T95924 | 02 | 3780 | 131 | 85 | 96 | D1- 90 | 3300 | - |
|  |  |  |  |  |  |  | C1- 15 | 480 |  |
|  |  |  |  |  |  |  | FL- 42.5 |  |  |
| M1079A1 | T93484 | 08 | 17379 | 263 | 96 | 100 | D1- 60 | 10775 | UN3166 |
|  |  |  |  |  |  |  | D2- 215 | 6604 |  |
| M142 | H53326 | 02 | 28560 | 312 | 96 | 105 | D1- 53 | 11424 | UN3166 |
|  |  |  |  |  |  |  | D2- 202 | 8568 |  |
|  |  |  |  |  |  |  | D3- 258 | 8568 |  |
| M1165A1 | T56383 | 03 | 9870 | 194 | 91 | 75 | D1- 26 | 5330 | UN3166 |
|  |  |  |  |  |  |  | D2- 156 | 4540 |  |

Chalk 1 (C-130H) will have the following:

1 x M1079A1 
1 x 463L Pallet 3250 LBS
10 Passengers w/ Floor loaded Bags.

-Load the remaining pax/cargo using **Chalk 2-3**

-All trailers will be hitched to a prime mover

-Pallets will be loaded aft of vehicles

-Baggage pallets always loaded closest to the ramp

-Adhere to all aircraft limitations (i.e., ACL, axle, compartment (width, height, weight), and passengers).

-Acknowledge all erroneous violations

Floor Loaded Bags Dimensions: # Pax = L” x W” x H”

2= 44’’ x 24” x 30’’                                3= 66” x 24” x 30”

4= 88” x 24” x 30”		            5 to 8= 88” x 48” x 30”

9 to 16= 88” x 48” x 60”                       17 to 19= 88” x 60” x 60”