FORMATIVE 3

Due to escalating conflict between the US and China, your unit has been tasked with deploying personnel, vehicles, and palletized equipment into Ulaanbaatar, a strategic location within the INDOPACOM theater. Using the following aircraft, C-130H/J, C-17, and C-5M, you will create a load plan to ensure the below personnel/equipment have been loaded IAW DTR, ATTLA, and Aircraft limitations.

Aircrafts (STD-AL)

- C-130H / 25,000

- C-130J / 40,000

- C-17 / 130,000

- C-5M / 150,000

Mission Type: Mobility

Chapter 3: Yes (75% Fuel)

Unit Airlift: TF CHARLIE

Departure Airfield: Osan AB (RKSO)

Destination: Chinggis Khaan Intl (ZMCK)

Passengers: 51 at 210 lbs each (2 minimum each chalk)

Passenger Bags: 190 LBS per passenger

-----------------------------------------------------------------------------------------

Your unit UMO has given you a compiled list of items that have been identified as needing to be deployed with the unit.  Create 1 cargo list with the following items.

| Description | LIN # | Lin Index # | Weight |
| --- | --- | --- | --- |
| Pallet, 463L x 23 | YA0038 | 01 | 4500 LBS |
| 3-PLT (T-3) x 1 | YA0048 | 01 | 12000 LBS |
| 2-PLT (T-2) x 1 | YA0046 | 01 | 710 LBS |
| M105A3 | W95811 | 98 | 2700 LBS |
| M1079A1 | T93484 | 08 | 18834 |
| M1165A1 x2 | T56383 | 03 | 9870 |
| M-ATV x 2 | M11442 | 01 | 32244 |
| M1145 ECV | YA0821 | 01 | 9800 |
| M977A2 | T59278 | 20 | 38865 |

Your unit has weighed and marked the equipment in your unit’s inventory that are identified to deploy.  Update the items in your cargo list according to the below dimensions and weights.

| Description | LIN # | Lin Index # | Weight | Length | Width | Height | Distance (IN) | Axles (lbs) | HAZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pallet, 463L x 13 | YA0038 | 01 | 7500 | 88 | 108 | 96 | - | - | - |
| Pallet, 463L x 2 | YA0038 | 01 | 4500 | 88 | 108 | 70 | - | - | - |
| Pallet, 463L x 2 | YA0038 | 01 | 3500 | 88 | 108 | 96 | - | - | - |
| Pallet, 463L x 5 | YA0038 | 01 | 5000 | 88 | 108 | 80 | - | - | - |
| Pallet, 463L | YA0038 | 01 | 4860 | 88 | 108 | 70 | - | - | - |
| 3-PLT (T-3) x 1 | YA0048 | 01 | 12000 | 268 | 108 | 70 | - | - | - |
| 2-PLT (T-2) x 1 | YA0046 | 01 | 2000 | 178 | 108 | 30 | - | - | - |
| M105A3 | W95811 | 98 | 2670 | 166 | 83 | 55 | D1- 115” | 2376 | - |
|  |  |  |  |  |  |  | C1- 18” | 294 |  |
|  |  |  |  |  |  |  | FL- 42” |  |  |
| M1079A1 | T93484 | 08 | 17379 | 263 | 96 | 102 | D1- 60” | 10775 | UN3166 |
|  |  |  |  |  |  |  | D2- 215” | 6604 |  |
| M1165A1 x 2 | T56383 | 03 | 9870 | 191 | 91 | 75 | D1- 26” | 5330 | UN3166 |
|  |  |  |  |  |  |  | D2- 156” | 4540 |  |
| M-ATV x 2 | M11442 | 01 | 32224 | 250 | 100 | 144 | D1- 38” | 14400 | UN3166 |
|  |  |  |  |  |  |  | D2- 182” | 17824 |  |
| M1145 ECV | YA0821 | 01 | 12000 | 235 | 101 | 102 | D1- 30” | 6000 | UN3166 |
|  |  |  |  |  |  |  | D2- 207” | 6000 |  |
| M977A2 | T59278 | 20 | 38865 | 401 | 140 | 112 | D1- 75” | 9717 | UN3166 |
|  |  |  |  |  |  |  | D2- 135” | 9716 |  |
|  |  |  |  |  |  |  | D3- 285” | 9716 |  |
|  |  |  |  |  |  |  | D4- 345” | 9716 |  |

**NOTE change symbol name for all M-ATVs from LGTK00072 to LGTK00054**

Chalk 1(C-5M) will have the following:

1 x M977A2 
1 x M1079A1
1 x M105A3
1 x 2,000 lbs 2 Pallet Train

1 x 12,000 lbs 3-Pallet Train

7 x 7,500 lbs 463L Pallets

3 x 5,000 lbs 463L Pallets
23 Passengers W/Bags.

-Load the remaining pax/cargo using **Chalk 2-4**

-All trailers will be hitched to a prime mover

-Pallets will be loaded aft of vehicles

-Baggage pallets always loaded closest to the ramp

-Adhere to all aircraft limitations (i.e. ACL, axle, compartment (width, height, weight), and passengers).

-Acknowledge all erroneous violations

Floor Loaded Bags Dimensions: # Pax = L” x W” x H”

2= 44’’ x 24” x 30’’                                3= 66” x 24” x 30”

4= 88” x 24” x 30”		            5 to 8= 88” x 48” x 30”

9 to 16= 88” x 48” x 60”                       17 to 19= 88” x 60” x 60”