(SLIDE 4)

Standardized System

1. Facilitates the movement of general cargo in and out of the aircraft

2. The system encompasses all aspects of cargo loading

a. Material handling equipment

b. Cargo loading platforms

c. Restraint equipment

d. In-aircraft systems

3. This system is the Air Force standard for movement of cargo

B.  Efficient system

1. Extremely efficient system, can reduce ground time by as much as 75 percent

2. Your first encounter with the system will probably be a 463L pallet

(SLIDE 5)

1. This system is permanently installed in all AMC cargo capable aircraft with the exception of the KC-135

a. The KC-135 can be fitted with a dual-rail system

b. The roller system must be requested prior to the aircraft leaving home station

(SLIDE 6)

2. Dual rail system consists of rows of rollers that allow easy movement of the palletized cargo into the aircraft

a. Many of these rollers are storable to convert the cargo deck to a flat, clear loading surface for wheeled cargo

b. Side rails guide the pallets into the aircraft and provide lateral and vertical restraint

(SLIDE 10)

2. 463L pallets have 22 steel tie down rings attached in such a manner that six rings are on each long side and five rings are on each short side

a. They have indents (notches) which are designed to accept the detent locks

b. Locks are located on numerous types of materials handling equipment and are found on board all airlift capable aircraft

Overall dimensions

1. 88 inches by 108 inches by 2 ¼-inch thick

a. Usable dimensions are-84 inches by 104 inches

b. This allows for 2 inches around the periphery of the pallet to attach straps, nets, or other restraint devices

2. An empty 463L pallet weighs 290 pounds

a. 355 pounds with nets

b. Maximum cargo carrying capacity of 10,000 pounds when using nets

c. 250 PSI limitation

3. The desired load capacity is 7,500 pounds to prolong pallet life

4. Pallets weighing less than 7,500 pounds can waste valuable aircraft space

(SLIDE 11)

1. Net sets

a. Two side nets

b. Usually green in color

c. Attaches to rings on pallet

2. One top net

a. Usually yellow in color

b. Attaches by hooks to the side net

d. A complete set of 463L nets (2 side nets and 1 top net) weighs 65 pounds

(SLIDE 18)

1. Normally, three pieces of dunnage are placed between stacks of ten pallets

2. Never stack empty pallets upside down, damage could occur

(SLIDE 19)

Protect the upper surface of the pallet from sharp edged cargo, place adequate shoring or cushioning materials between the cargo and the pallet

Be careful when transporting 463L pallets with a forklift, the tine tips can easily damage the pallet surface making it unusable

Never push or slide 463L pallets across any solid surface, to do so will cause damage to the skin

(SLIDE 20)

Protect nets from adverse climatic conditions

1. Netting materials will mildew and deteriorate

2. The metal hooks will also rust if not properly cared for

3. Hang and dry all nets after use

4. Never pile wet nets together for storage

(SLIDE 21)

Ensure the pallet is level and not warped

(SLIDE 23)

For more extensive damage (such as bent rails or damaged/missing tiedown rings), the pallet will not be accepted for air shipment

(SLIDE 24)

It must be sent to depot for repairs

(SLIDE 25)

Keep pallets as clean as possible to protect the cargo and prevent the spread of insects, dirt born infestations, etc

(SLIDE 27)

2. Place lighter and/or smaller items on top or along the side of the heavier cargo

3. Construct the load in a square or pyramid shape when possible

a. Do not allow cargo to extend beyond the vertical stacking line (84 by 104 inches) of netted pallets

b. This makes the load stable, easy to handle, easy to secure to the pallet, and prevents binding in the aircraft 463L dual rail system

(SLIDE 30)

A pallet build up procedures checklist will be discussed during the hands on pallet build up and weighing and marking exercise

(SLIDE 31)

1. Exceeding Pallet Dimensions

a. When there is a requirement to ship cargo that exceeds these dimensions, we may join two or more pallets together

1. Referred to as the marrying of pallets or pallet trains (double or two)

2. The same applies to three, four, five, or six pallets when joined

b. Before marrying pallets, inspect them to be sure they are clean and in good repair

1. Ensure the rails are not bowed and the tiedown rings are securely mounted

(SLIDE 36)

Use barrier boards and chain bridles for loose heavy items such as lumber and pipe

If height exceeds 30 inches, apply multiple chain bridles

(SLIDE 37)

Lateral restraint must be symmetrical

(SLIDE 38)

a. More than ½ of the required restraint applied to the axles will require the vehicle’s suspension to assume the majority of restraint

b. This could result in suspension failure and loose equipment in-flight