Orqa Controller

Simulation Calibration Instructions

- Select “Options” on home screen of Liftoff.

- Select “Controls” and the “Controller”.

- Select “Calibrate” and then “Start Calibration”.

- Move the gimbals in the instructed method returning them to center each time.

- When complete select “Save” and then return to home screen using the “Exit: button on the bottom left.

- Select “Free Flight”.

- Select the level “Straw Bales”.

- Select “No Track”.

- Select drone “Rotor Riot CL1”.

- Lower the camera angle to 10-20 degrees using the down arrow on the keyboard.

- Lower the throttle stick all the way down.

- Arm the drone using the left trigger button.

- Slowly increase throttle until you are lifting into the air. Do not move the right stick or Yaw.

- Your drone should not yaw, pitch, or roll in any direction. If it does not, then you are done.

- If your drone moves in any direction take note of the direction of the yaw, pitch, or roll observed.

- Exit back to the main menu.

- If you had yaw, pitch, or roll return to the calibration menu using steps 1-2.

- Select “Fine-tune”.

- Make minor adjustments to the line you were experiencing. Suggest starting with .03.

- Click save and return to “Straw Bales” using steps 6-9 and try again.

- Continue making minor adjustments until the drone does not yaw, pitch, or roll when rising directly up in the air.