FPV Drone Pre-Flight Checklist

I. Pilot Preparation & Mental State

- Rested and Focused: Are you well-rested and free from distractions? Avoid flying if feeling unwell, stressed, or rushed.

- Clear Head: No alcohol or drugs in your system.

- Knowledge of Regulations: Are you aware of and compliant with local drone regulations (e.g., airspace restrictions, VLOS requirements, weight limits)?

- Emergency Plan: Have you thought about what you would do in various emergency scenarios (e.g., lost video, motor failure, lost signal)?

II. Drone (Quad) Inspection

- Propellers:

  - Are all props securely attached?

  - Are they free from nicks, cracks, or bends?

  - Are they installed in the correct direction (CW/CCW for each motor)?

  - Are they the correct pitch/size for your motors?

- Motors:

  - Spin freely by hand?

  - No excessive play in the bells?

  - No debris or wires touching the motor bells?

  - Motor screws secure?

- Frame:

  - No visible cracks or damage to carbon fiber or other frame components?

  - All standoffs and screws tight?

  - Antenna mounts secure?

- Electronics (General):

  - All wires neatly tucked away and secured (zip ties, heat shrink)? No loose wires that could foul props or short out.

  - No visible shorts or exposed solder joints?

  - FC, ESCs, VTX, RX, camera, and GPS (if applicable) securely mounted?

  - Heat shrink/electrical tape on exposed connections as needed.

- Battery Strap:

  - Secure and correctly routed through the frame.

  - No signs of wear or fraying.

III. Lipo Battery (Drone)

- Voltage Check: Each cell is at storage voltage or fully charged (depending on intent) and balanced. Use a LiPo checker.

- Physical Condition: No puffing, tears in the wrapper, or damaged balance leads/main connector.

- Connector: XT60/XT30/etc. connector is clean and free of corrosion.

- Temperature: Battery is at ambient temperature (not hot or cold from storage/charging).

IV. Radio Transmitter (TX)

- Battery Level: Sufficient charge for the planned flights.

- Antenna: Securely attached and oriented correctly (if applicable).

- Model Selection: Correct drone model selected on the TX.

- Switches: All switches in their correct "off" or "disarmed" positions.

- Stick Calibration/Trim: (Quick check if recently calibrated or if issues suspected).

- Range Test: (If applicable and within a safe, controlled environment).

- Control Link: Ensure the TX is bound to the drone's receiver.

V. FPV Goggles/Screen

- Battery Level: Sufficient charge.

- Antennas: Securely attached to the goggle module (if applicable).

- Lens/Screen Clean: Free of smudges or dust.

- Focus/Diopter Adjustment: (If applicable) adjusted for clear vision.

- DVR: (If applicable) SD card inserted and formatted.

VI. Pre-Flight Setup (Before Plugging in Drone Battery)

- Arming Method Confirmed: Understand how you will arm and disarm.

- Fail-safe Checked: (Crucial! Done periodically, not necessarily every flight). Ensure your fail-safe is set up correctly (e.g., motors off, land, return to home if GPS).

- Arming Switch Test (Optional): Ensure arming switch functions before props are on.

- VTX Checked: Ensure VTX is properly programmed with transmission frequency and power settings.

VII. Environmental Check

- Location:

  - Are you away from restricted airspace (airports, military bases, no-fly zones)?

  - Are there any overhead power lines, trees, or other potential hazards?

- Weather:

  - Wind speed and direction: Is it safe to fly in current wind conditions for your drone?

  - Precipitation: No rain, snow, or excessive moisture.

  - Visibility: Clear line of sight to the drone.

  - Temperature: Extreme heat or cold can affect battery and electronics.

- Sun Position: Will the sun be in your eyes or glaring on your goggles during flight?

- Interference: Are there any known sources of RF interference (e.g., cell towers, high-power antennas) nearby?

VIII. Final Steps (Just Before Arming)

- Drone on Stable Surface: Ensure the drone is on a level, clear surface, away from anything it could hit upon arming.

- Goggles On / Screen On: Confirm clear video feed.

- Clear of Props: Ensure fingers, clothing, and anything else are well clear of propellers.

- One Last Look: Quickly scan the drone again for anything obviously out of place.

Post-Flight (Brief Check-in):

- Inspect drone for damage after landing.

- Disconnect battery immediately.

- Allow motors/ESCs to cool if hot.

- Discharge LiPo to storage voltage if not flying again soon.