EMERGENCY Procedures

Loss of Video Signal

Level pitch

Switch to angle mode and release right stick inputs

Increase throttle to climb

Maintain throttle increase for a ‘six-thousand’ count or until video is regained

If video is regained, reorient yourself and check battery voltage

Continue mission

If video is not regained

Activate failsafe

Annotate aircrafts last known position

Loss of flight control / RX signal

Switch to angle mode

Input left and right roll inputs until RX is regained

If control is not regained

Activate failsafe

Annotate aircrafts last known position

Loss of situational awareness

Angle mode

Increase altitude

Identify terrain features to reorient

Interrupted flight control or video in a tactical environment (denied, degraded, disrupted space operational environment [D3SOE])

Upon RX loss noted, input left and right Roll commands

Upon RX recovery

Maintain altitude and utilize a clover leaf maneuver every 400m in an attempt to locate signal jammers

If unable to identify a route, but signal jammer can be identified, apply knowledge of antenna theory to exploit gaps in jamming antenna

INSERT BETAFLIGHT FAILSAFE SNIPS. TRAINING IS LAND IN PLACE AND TACTICAL IS FORWARD FLIGHT.

INSERT ANTENNA THEORY PICTURES