SPECIAL INSTRUCTIONS

- This is a hands-on practical exercise.  You will use only the reference publications shown.  You may use notes.  Do not talk to other students at any time.  Speak only to the instructor.

- Be sure to read the “SITUATION”.  If you have any questions, ask the instructor now.

- The soldier will score a GO (G) if all steps are passed.  The soldier will score a NO-GO (NG) if any steps are failed.  If the soldier scores a NG on any step, the soldier will be shown what was done incorrectly, how to do it properly, and then will be re-tested.

- During this practical exercise you will need:

  - AR 25-400-2

  - DA Pam 750-8

  - TM 9-1005-319-10

  - TM 9-1005-319-23&P

  - Computer with forms program or blank forms.

  - Safety Goggles

  - Rifle 5.56mm, M16A2

  - Magazine

  - Tool Kit Small Arms Repairman

  - Pencil or Pen

GENERAL INFORMATION

| Your unit of assignment | D Co, 13th MP BN Fort Lee, VA 23801 |
| --- | --- |
| Unit DODAAC | WK4C30 |
| Unit UIC | WBAKD0 |
| Company Commander | Paul J. Sfransky CPT, MP |
| Armorer/Supply Clerk | You are the Armorer/Supply Clerk |
| Unit Phone Number | 765-9185 |
| Maintenance Supervisor | SFC Mike McGuire |
| Direct Support Unit (DSU) | 581st Maintenance Company |
| M16A2 National Stock Number | 1005-01-128-9936 |
| M16A2 Serial Number | Student use SN from actual weapon. |
| M16A2 EIC | 4GM |

GENERAL SITUATION

You are the unit armorer for D Co 13th MP BN.  The Supply Sergeant has informed you that the M16A2 Rifles are scheduled for their quarterly service.  You have been tasked with performing organizational maintenance on the 5.56mm, M16A2 Rifles assigned to the unit.  Today’s date is 22 October 20XX.

Requirement 1

Using TM 9-1005-319-10, TM 9-1005-319-23&P, and the performance sheet listed below, unload and clear the 5.56mm Rifle, M16A2.

| UNLOAD AND CLEAR THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Point weapon in a safe direction. |  |  |
| - Place selector lever on SAFE. If weapon is not cocked, the lever cannot be placed in the SAFE position. |  |  |
| - Remove the magazine. |  |  |
| - To lock bolt open, pull the charging handle rearward. |  |  |
| - Press the bottom of the bolt catch and allow the bolt to move forward until it engages the catch. |  |  |
| - Return charging handle to forward position. |  |  |
| - Place the selector lever on SAFE. |  |  |
| - Check the receiver and chamber to ensure there is NO ammo. |  |  |
| - With the selector lever pointing toward SAFE, allow the bolt to go forward by pressing the upper portion of the bolt catch. |  |  |

Requirement 2

Using TM 9-1005-319-10, TM 9-1005-319-23&P and performance sheet listed below, disassemble the M16A2 Rifle.

| DISASSEMBLY OF THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Remove sling. |  |  |
| - Remove handguards. |  |  |
| - Separate upper receiver from lower receiver. |  |  |
| - Remove charging handle and bolt carrier. |  |  |
| - Remove firing pin retaining pin. |  |  |
| - Remove firing pin. |  |  |
| - Remove bolt cam pin. |  |  |
| - Remove bolt assembly from carrier. |  |  |
| - Remove extractor pin. |  |  |
| - Remove extractor and spring. |  |  |
| - Remove buffer and action spring. |  |  |

STUDENT:  You must stop at this time and have an instructor verify that you have correctly completed requirements 1 and 2.

Requirement 3

Using TM 9-1005-319-23&P, DA Pam 750-8, and a computer with an Army form program or blank DA Form 5598-E (Figure 1), and DA Form 5990-E (Figure 2); prepare a DA Form 5598-E and verify the DA Form 5590-E on the M16A2 Rifle.  Annotate all corrective actions as required.  The following deficiencies were found on the rifle. (Instructor may stage or add deficiencies). Today’s date is 22 Oct 20XX.  Identify any inaccuracies on the DA Form 5990-E by lining through the inaccurate information and writing in the correct information.

DEFICIENCY

Heatshield rattles when installed on weapon

Weak extractor spring

Requirement 4

Using TM 9-1005-319-10, TM 9-1005-319-23&P, and the performance sheet listed below, student will reassemble the M16A2 Rifle.

| REASSEMBLY OF THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Insert action spring and buffer. |  |  |
| - Insert extractor and spring. |  |  |
| - Push in extractor pin. |  |  |
| - Slide bolt into carrier. |  |  |
| - Replace bolt cam pin. |  |  |
| - Drop in and seat. |  |  |
| - Pull bolt out. |  |  |
| - Replace firing pin and firing pin retaining pin |  |  |
| - Engage, then push charging handle part way. |  |  |
| - Slide in bolt carrier group. |  |  |
| - Push charging handle and bolt carrier group together. |  |  |
| - Join upper and lower receivers. |  |  |
| - Engage receiver pivot pin. |  |  |
| - Close upper and lower receiver groups. |  |  |
| - Push in takedown pin. |  |  |
| - Replace hand guards (use the Buddy System). |  |  |
| - Replace sling. |  |  |

Requirement 5

Using TM 9-1005-319-10, TM 9-1005-319-23&P, and the performance sheet listed below, perform a functions check on the M16A2 Rifle.

| FUNCTION CHECK | GO | NO GO |
| --- | --- | --- |
| WARNING: Ensure that the rifle is clear before performing a function check. |  |  |
| SAFE |  |  |
| - Pull charging handle to the rear and release. |  |  |
| - Place selector lever on SAFE. Pull trigger to the rear, the hammer should not fall |  |  |
| SEMI |  |  |
| - Move selector lever to SEMI. |  |  |
| - Pull trigger and hold it to the rear, the hammer should fall. |  |  |
| - Pull charging handle to the rear and then release. |  |  |
| - Release trigger and pull again, the hammer should fall. |  |  |
| BURST |  |  |
| - Rotate selector lever to BURST. |  |  |
| - Pull charging handle to the rear and release. |  |  |
| - Pull trigger to the rear and hold to the rear, hammer should fall. |  |  |
| - Pull charging handle to the rear and release three times. |  |  |
| - Release trigger and squeeze trigger to the rear again, hammer should fall. |  |  |

STUDENT:  You must stop at this time and have an instructor verify that you have correctly completed requirements 4 and 5.

| PERFORMANCE MEASURE | RESULTS |  |
| --- | --- | --- |
| - Clear the rifle before disassembly | P | F |
| - Disassemble the rifle. | P | F |
| - Prepare DA Form 5598-E and verify the DA Form 5990-E | P | F |
| - Assemble the rifle. | P | F |
| - Perform a function check | P | F |
| - Weapon functions as mission-ready | P | F |
| Verified By Signature: |  |  |

| DATE: 20XX1020 | EQUIPMENT MAINTENANCE AND | DA FORM 5988-E |
| --- | --- | --- |
|  | INSPECTION WORKSHEET |  |
| WBAKD0 | D Co, 13TH MP BN |  |

-------------------------------------------------------------- EQUIPMENT DATA ---------------------------------------------------------------

| ADMIN NUM: DXX | EQUIP SERIAL NUMBER: XXXXXXX |
| --- | --- |
| EQUIP MODEL: M16A2 | REGISTRATION NUMBER: |
| EQUIP NOUN: Rifle 5.56mm | TYPE INSPECTION: Quarterly |
| EQUIP NSN: 1005011289936 | CURRENT READING: |

| NUMBER | DATE | CHANGE NUMBER |
| --- | --- | --- |
| PUBLICATION: TM 9-1005-319-10 | 10/98 |  |
| PUBLICATION: TM 9-1005-319-23&P | 11/08 |  |
| INSPECTORS LIC#: ____________ TIME: _________ | SIGNATURE: _____________________ TIME: _____ |  |

-------------------------------------------------------------- PARTS REQUESTED --------------------------------------------------------------

| FAULT | DOC | NUM | NIIN | NOUN | QTY DUE/REC | STATUS DATE | DATE COMP | P D R L I C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

------------------------------------------------------------ MAINTENANCE FAULTS ------------------------------------------------------------

| ITEM NUM | FAULT DATE | FAULT STATUS | FAULT DESCRIPTION | CORRECTIVE ACTION | OPER HRS LIC# |
| --- | --- | --- | --- | --- | --- |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |

Figure 1

| DATE: 20XX1024 | MAINTENANCE REQUEST | DA FORM 5990-E |
| --- | --- | --- |

---------------------------------------------------------------- CUSTOMER DATA ----------------------------------------------------------------

| UIC: WK4C30 | D Co, 13TH MP BN | PHONE: 765-9185 |
| --- | --- | --- |
| UTIL CODE: 0 |  |  |

----------------------------------------------------------------- ACTIVITY DATA -----------------------------------------------------------------

| SUP WON: | 581ST MAINTENANCE CO | PHONE: 734-7235 |
| --- | --- | --- |
| SUP UIC: WBBDXF |  | SHOP SEC: . |

--------------------------------------------------------------- EQUIPMENT DATA ---------------------------------------------------------------

| TYPE MNT REQ: 3 | ID: A NSN: 1005011289936 | MODEL: M16A2 . |
| --- | --- | --- |
| NOUN: Rifle 5.56mm | SER NUM: XXXXXXX | QTY: 00001 . |
| ORG WON : | PRIORITY: 05 . | FAILURE DETECTED: A . |
| MI/KM: ------------ | HOURS: ----- | ROUNDS: . |
| IN WARRANTY: | LEVEL OF WORK: F . | ADMIN NUM: DXX . |
| DEFICIENCY: Weak Extractor Spring |  |  |
| PD AUTHENTICATING SIGNATURE: ___________________________________________________________ |  |  |

-------------------------------------------------------------- SIGNATURE DATA ------------------------------------------------------------------

| SUBMITTED BY: ______________________________ |  | ORD DATE: ___________ | MIL TIME: ___________ |
| --- | --- | --- | --- |
| ACCEPTED BY: ___________________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |

------------------------------------------------------------------ ACTION DATA ------------------------------------------------------------------

| WORK STARTED BY: _____________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |
| --- | --- | --- | --- |
| INSPECTED BY: ______________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |
| PICKED UP BY: _____________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |

--------------------------------------------------------------- COMPLETION DATA --------------------------------------------------------------

| QTY RPR: _____________ | QTY CONDEMNED: ___________ |  |  | NRTS: ___________ |  |
| --- | --- | --- | --- | --- | --- |
| EVAC WON: ______________________ |  |  | EVAC UNIT NAME: _______________________ |  |  |

Figure 2