SPECIAL INSTRUCTIONS

- This is a hands-on practical exercise.  You will use only the reference publications shown.  You may use notes.  Do not talk to other students at any time.  Speak only to the instructor.

- Be sure to read the “SITUATION”.  If you have any questions, ask the instructor now.

- The soldier will score a GO (G) if all steps are passed.  The soldier will score a NO-GO (NG) if any steps are failed.  If the soldier scores a NG on any step, the soldier will be shown what was done incorrectly, how to do it properly, and then will be re-tested.

- During this practical exercise you will need:

  - Safety Goggles

  - M107 Long Range Sniper Rifle

  - 10 Ea A560 – Dummy Cartridge Cal .50 Single Round

  - Tool Kit Small Arms Repairman

  - AR 25-400-2

  - TM 9-1005-239-10

  - TM 9-1005-239-23&P

  - DA Pam 750-8

  - Pencil or Pen

GENERAL INFORMATION

| Your unit of assignment | D Co, 13th MP BN Fort Lee, VA 23801 |
| --- | --- |
| Unit DODAAC | WK4C30 |
| Unit UIC | WBAKD0 |
| Company Commander | Paul J. Sfransky CPT, MP |
| Supply Clerk/Assistant Armorer | You are the Supply Clerk/Assistant Armorer |
| Unit Phone Number | 765-9185 |
| Maintenance Supervisor | SFC Mike McGuire |
| Direct Support Unit (DSU) | 581st Maintenance Company |
| M107 National Stock Number | 1005-01-469-2133 |
| M107 Serial Number | Use serial number form weapon |
| M107 EIC | 4HA |

GENERAL SITUATION 1

You are the armorer for D Co 13th MP BN.  The Commander has informed you that the M107 Sniper Rifles are scheduled for their quarterly service.  You have been tasked with performing organizational maintenance on the M107 Long Range Sniper Rifle.

Requirement 1

Using TM 9-1005-239-10, TM 9-1005-239-23&P, and the performance sheet listed below, unload and clear the M107 Long Range Sniper rifle.

| UNLOAD AND CLEAR THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Place the weapon safety on “SAFE”. Press the magazine catch forward towards the magazine, and remove the magazine. |  |  |
| - Pull the charging handle to the rear, which will eject any cartridge still chambered. |  |  |
| - After the rifle is unloaded, and with the charging handle to the rear, look into the chamber make certain the breech area and chamber are empty |  |  |

Requirement 2

Using TM 9-1005-239-10, TM 9-1005-239-23&P and performance sheet listed below, field strip the M107 Long Range Sniper Rifle and locate and describe the major components.

| DISASSEMBLY OF THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Ensure that the weapon is unloaded and on SAFE before performing the disassembly. |  |  |
| - Remove rear lock and midblock pins. |  |  |
| - Grasp charging handle on the bolt carrier and pull to the rear until the bolt clears the barrel extension. |  |  |
| - While holding the charging handle to the rear, lift the back end of the upper receiver assembly until it clears the bolt. Slowly allow the bolt carrier to return to its forward position. |  |  |
| - Disengage front hook from the front hook pin on the lower receiver and lift upper receiver assembly clear of lower receiver assembly. |  |  |
| - Pull the bolt carrier rearward and insert the rear lock pin through the buffer and buffer spring |  |  |
| - Gently pull the bolt carrier forward and lift out the lower receiver assembly. |  |  |

Requirement 3

Using TM 9-1005-239-23&P, DA Pam 750-8, DA Form 5988-E (Figure 1) and DA Form 5990-E (Figure 2); complete the DA Form 5988-E and verify the DA Form 5990-E on the LRSR M107.  Annotate all corrective actions as required.  Today’s date is 15 April 20XX.  The monopod foot is stocked in unit PLL and was replaced.  The following deficiencies were found on the rifle. (Instructor can stage or add additional deficiencies.)  If inaccuracies are found on the DA Form 5990-E, line thru the inaccurate information and write the correct information.

DEFICIENCIES FOUND

Unserviceable Monopod Foot

Cracked Muzzle Brake

Requirement 4

Using TM 9-1005-239-10, TM 9-1005-239-23&P, and the performance sheet listed below, student will reassemble the M107 LRSR.

| REASSEMBLY OF THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Place bolt carrier assembly into forward part of lower receiver assembly |  |  |
| - Pull bolt and carrier assembly rearward and carefully remove rear lock pin from mainspring buffer. |  |  |
| - Align front hook of upper receiver assembly with front hook pin on lower receiver assembly. |  |  |
| - With upper receiver assembly in position on lower receiver assembly, secure with rear lock pin and midblock pin. |  |  |
| - Insert cartridge magazine into lower receiver assembly. |  |  |

| PERFORMANCE MEASURE | RESULTS |  |
| --- | --- | --- |
| - Clear the rifle before disassembly | P | F |
| - Disassemble the rifle. | P | F |
| - Prepare DA Form 5598-E and DA Form 5990-E | P | F |
| - Assemble the rifle. | P | F |
| - Perform a function check | P | F |
| - Weapon functions as mission-ready | P | F |
| Verified By Signature: |  |  |

| DATE: 20XX0410 | EQUIPMENT MAINTENANCE AND | DA FORM 5988-E |
| --- | --- | --- |
|  | INSPECTION WORKSHEET |  |
| WBAKD0 | D Co, 13TH MP BN |  |

-------------------------------------------------------------- EQUIPMENT DATA ---------------------------------------------------------------

| ADMIN NUM: DXX | EQUIP SERIAL NUMBER: XXXXXXX |
| --- | --- |
| EQUIP MODEL: M107 | REGISTRATION NUMBER: |
| EQUIP NOUN: Long Range Sniper Rifle, Cal .50 | TYPE INSPECTION: Quarterly |
| EQUIP NSN: 1005014692133 | CURRENT READING: |

| NUMBER | DATE | CHANGE NUMBER |
| --- | --- | --- |
| PUBLICATION: TM 9-1005-239-10 | 11/10 |  |
| PUBLICATION: TM 9-1005-239-23&P | 09/10 |  |
| INSPECTORS LIC#: __S1234_____ TIME: _________ | SIGNATURE: _____________________ TIME: _____ |  |

-------------------------------------------------------------- PARTS REQUESTED --------------------------------------------------------------

| FAULT | DOC | NUM | NIIN | NOUN | QTY DUE/REC | STATUS DATE | DATE COMP | P D R L I C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

------------------------------------------------------------ MAINTENANCE FAULTS ------------------------------------------------------------

| ITEM NUM | FAULT DATE | FAULT STATUS | FAULT DESCRIPTION | CORRECTIVE ACTION | OPER HRS LIC# |
| --- | --- | --- | --- | --- | --- |
| _2__ | 15 Apr XX | __/__ | Monopod Foot Unserv_________ | Replaced Monopod Foot 1005-01-465-5702____________ | __ S1234 |
| _4__ | 15 Apr XX | __X__ | Cracked Muzzle Brake_________ | DA Form 5990-E (SPT MNT)____ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |

Figure 1

| DATE: 20XX0418 | MAINTENANCE REQUEST | DA FORM 5990-E |
| --- | --- | --- |

---------------------------------------------------------------- CUSTOMER DATA ----------------------------------------------------------------

| UIC: WK4C30 WBAKD0 | D Co, 13TH MP BN | PHONE: 765-9185 |
| --- | --- | --- |
| UTIL CODE: 0 |  |  |

----------------------------------------------------------------- ACTIVITY DATA -----------------------------------------------------------------

| SUP WON: | 581ST MAINTENANCE CO | PHONE: 734-7235 |
| --- | --- | --- |
| SUP UIC: WBBDXF |  | SHOP SEC: . |

--------------------------------------------------------------- EQUIPMENT DATA ---------------------------------------------------------------

| TYPE MNT REQ: K 1 | ID: A NSN: 1005014692133 | MODEL: M107 . |
| --- | --- | --- |
| NOUN: LRSR, Cal .50 | SER NUM: XXXXXXX | QTY: 00001 . |
| ORG WON : | PRIORITY: 05 . | FAILURE DETECTED: A . |
| MI/KM: ------------ | HOURS: ----- | ROUNDS: . |
| IN WARRANTY: | LEVEL OF WORK: F . | ADMIN NUM: DXX . |
| DEFICIENCY: Cracked Muzzle Brake |  |  |
| PD AUTHENTICATING SIGNATURE: _Paul J. Sfransky________________________________________ |  |  |

-------------------------------------------------------------- SIGNATURE DATA ------------------------------------------------------------------

| SUBMITTED BY: ______________________________ |  | ORD DATE: ___________ | MIL TIME: ___________ |
| --- | --- | --- | --- |
| ACCEPTED BY: ___________________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |

------------------------------------------------------------------ ACTION DATA ------------------------------------------------------------------

| WORK STARTED BY: _____________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |
| --- | --- | --- | --- |
| INSPECTED BY: ______________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |
| PICKED UP BY: _____________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |

--------------------------------------------------------------- COMPLETION DATA --------------------------------------------------------------

| QTY RPR: _____________ | QTY CONDEMNED: ___________ |  |  | NRTS: ___________ |  |
| --- | --- | --- | --- | --- | --- |
| EVAC WON: ______________________ |  |  | EVAC UNIT NAME: _______________________ |  |  |

Figure 2