SPECIAL INSTRUCTIONS

- This is a hands-on practical exercise.  You will use only the reference publications shown.  You may use notes.  Do not talk to other students at any time.  Speak only to the instructor.

- Be sure to read the “SITUATION”.  If you have any questions, ask the instructor now.

- The soldier will score a GO (G) if all steps are passed.  The soldier will score a NO-GO (NG) if any steps are failed.  If the soldier scores a NG on any step, the soldier will be shown what was done incorrectly, how to do it properly, and then will be re-tested.

- During this practical exercise you will need:

  - Safety Goggles

  - M240B Machine Gun

  - M240B Machine Gun Layout Chart

  - M172 Linked Dummy Rounds

  - Tool Kit Small Arms Repairman

  - AR 25-400-2

  - TM 9-10005-313-10

  - TM 9-1005-313-23&P

  - DA Pam 750-8

  - Computer with forms program or blank forms.

  - Pencil or Pen

GENERAL INFORMATION

| Your unit of assignment | D Co, 13th MP BN Fort Lee, VA 23801 |
| --- | --- |
| Unit DODAAC | WK4C30 |
| Unit UIC | WBAKD0 |
| Company Commander | Paul J. Sfransky CPT, MP |
| Armorer/Supply Clerk | You are the Armorer/Supply Clerk |
| Unit Phone Number | 765-9185 |
| Maintenance Supervisor | SFC Mike McGuire |
| Direct Support Unit (DSU) | 581st Maintenance Company |
| M240B Machine Gun National Stock Number | 1005-01-412-3129 |
| M240B Machine Gun Serial Number | Use the serial number from weapon |
| M240B Machine Gun EIC | 4BJ |

GENERAL SITUATION

You are the unit armorer for D Co 13th MP BN.  The Company Commander has informed you that the M240B Machine Guns are scheduled for their quarterly service.  You are to perform field maintenance on the 7.62mm Machine Gun, M240B.

Requirement 1

Using TM 9-1005-313-10, TM 9-1005-313-23&P, and the performance sheet listed below, unload and clear the M240B Machine Gun.

| UNLOAD AND CLEAR THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Place safety to “F”. |  |  |
| - Pull the cocking handle assembly to the rear. |  |  |
| - Lock the bolt to the rear. |  |  |
| - Return cocking handle to the forward position. |  |  |
| - Place safety to “S” |  |  |
| - Push in latches to open cover assembly. |  |  |
| - Remove ammo belt. |  |  |
| - Raise feed tray. |  |  |
| - Look into chamber to make sure no round is chambered. |  |  |
| - Lower feed tray. |  |  |
| - Place safety to “F”. |  |  |
| - Hold cocking handle assembly to rear, squeeze trigger, and ease bolt forward to close and lock. |  |  |

Requirement 2

Using TM 9-1005-313-10, TM 9-1005-313-23&P and performance sheet listed below, disassemble the M240B Machine Gun.

| DISASSEMBLY OF THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Depress barrel locking latch and hold. |  |  |
| - Turn barrel release/carrying handle to upright position. CAUTION: Be sure bolt is forward or receiver can be damaged. |  |  |
| - Remove barrel. Push forward and lift barrel out. |  |  |
| - Remove heat shield assembly from barrel. Lift rear of heat shield assembly from barrel. Lift rear of heat shield assembly off barrel then pry front tabs out of hole bushing. |  |  |
| - Depress spring and remove spring pin. |  |  |
| - Pull trigger-housing assembly down and back. |  |  |
| - Depress back plate latch and lift buffer/buttstock assembly straight up. |  |  |
| - Press driving spring in, up, and pull out. |  |  |
| - Depress cover latches and raise cover assembly. |  |  |
| - Pull cocking handle assembly back; then pull bolt and operating rod assembly out. |  |  |
| - Close cover. Push out spring pin as far as possible with the back of buffer. |  |  |
| - Then remove pin with fingers. |  |  |
| - Depress cover latches; lift upward and remove cover assembly. |  |  |
| - Remove feed tray. |  |  |
| - Extend bipod legs to down and locked position. |  |  |
| - Pull handguard straight down and off gas cylinder. |  |  |

STUDENT:  You must stop at this time and have an instructor verify that you have correctly completed requirements 1 and 2.

Requirement 3

Using TM 9-1005-313-23&P, DA Pam 750-8, a DA Form 5988-E (Figure 1), and DA Form 5990-E (Figure 2); complete the DA Form 5988-E for the Quarterly service and verify the DA Form 5990-E on the M240B Machine Gun.  Annotate all corrective actions as required on the DA Form 5988-E.  When verifying the DA Form 5990-E, line thru any inaccuracies and write in the correct information. Today’s date is 20 April 20XX.

The following deficiencies were found on the weapon. (Instructor can stage or add additional faults)

DEFICIENCY

Feed Tray Cracked

Requirement 4

Using TM 9-1005-313-10, TM 9-1005-313-23&P, and the performance sheet listed below, student will reassemble the M240B Machine Gun.

| REASSEMBLY OF THE WEAPON | GO | NO GO |
| --- | --- | --- |
| - Position feed tray and cover assembly; push cover assembly forward; close cover and insert spring pin. NOTE: Insert spring pin from right side. |  |  |
| - Open cover assembly. Be sure cover detent holds cover assembly open. |  |  |
| - Set bolt and operating rod assembly on top of rails (receiver). Extend bolt to unlocked position, and then push the assembly all the way in the receiver. Close cover assembly and lock. |  |  |
| - Insert driving spring into operating rod assembly. Push it in fully and lower it to seat the stud in hole of the receiver. |  |  |
| - Install buffer assembly/buffer and spade assembly/buttstock and buffer assembly and make sure it latches. NOTE: Be sure operating rod assembly is properly seated in the receiver before operating the weapon. Top of buffer/buffer and spade grip/buttstock should be flush with top if receiver. |  |  |
| - Insert spring pin. NOTE: For M240B insert pin from right side only. |  |  |
| - Snap handguard over gas cylinder making sure tabs are to the front of the weapon. WARNING: Before firing, make sure the barrel is locked tightly in the receiver. If the barrel is not locked tight, threads in receiver could be damaged or cause personal injury. |  |  |
| NOTE: Barrel release must be in upright position when installing barrel. - Insert barrel fully into socket and push barrel release/barrel carrying handle to the right as far as it will go (fewer than 2 or more than 7 clicks indicate defective parts) to lock |  |  |

Requirement 5

Using TM 9-1005-313-10, TM 9-1005-313-23&P, and the performance sheet listed below, perform a functions check on the M240B Machine Gun.

| FUNCTION CHECK | GO | NO GO |
| --- | --- | --- |
| WARNING: Ensure that the machine gun is clear before performing a function check. |  |  |
| - Place safety to “F” |  |  |
| - Pull the cocking handle to the rear to lock bolt back. |  |  |
| - Return the cocking handle forward to the locked position. |  |  |
| - Place safety to “S” |  |  |
| - Depress trigger; nothing should happen. |  |  |
| - Place safety to “F”. |  |  |
| - Hold cocking handle assembly to rear, depress trigger, and ease bolt forward to close and lock. |  |  |

STUDENT:  You must stop at this time and have an instructor verify that you have correctly completed requirements 4 and 5.

| PERFORMANCE MEASURE | RESULTS |  |
| --- | --- | --- |
| - Clear the weapon before disassembly | P | F |
| - Disassemble the weapon. | P | F |
| - Prepare DA Form 5988-E and DA Form 2407 | P | F |
| - Assemble the weapon. | P | F |
| - Perform a function check | P | F |
| - Weapon functions as mission-ready | P | F |
| Verified By Signature: |  |  |

| DATE: 20XX0417 | EQUIPMENT MAINTENANCE AND | DA FORM 5988-E |
| --- | --- | --- |
|  | INSPECTION WORKSHEET |  |
| WBAKD0 | D Co, 13TH MP BN |  |

-------------------------------------------------------------- EQUIPMENT DATA ---------------------------------------------------------------

| ADMIN NUM: DXX | EQUIP SERIAL NUMBER: XXXXXXX |
| --- | --- |
| EQUIP MODEL: M240B | REGISTRATION NUMBER: |
| EQUIP NOUN: Machine Gun 7.62mm | TYPE INSPECTION: Quarterly |
| EQUIP NSN: 1005014123129 | CURRENT READING: |

| NUMBER | DATE | CHANGE NUMBER |
| --- | --- | --- |
| PUBLICATION: TM 9-1005-313-10 | 08/10 | 7 |
| PUBLICATION: TM 9-1005-313-23&P | 08/10 | 3 |
| INSPECTORS LIC#: ____________ TIME: _________ | SIGNATURE: _____________________ TIME: _____ |  |

-------------------------------------------------------------- PARTS REQUESTED --------------------------------------------------------------

| FAULT | DOC | NUM | NIIN | NOUN | QTY DUE/REC | STATUS DATE | DATE COMP | P D R L I C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

------------------------------------------------------------ MAINTENANCE FAULTS ------------------------------------------------------------

| ITEM NUM | FAULT DATE | FAULT STATUS | FAULT DESCRIPTION | CORRECTIVE ACTION | OPER HRS LIC# |
| --- | --- | --- | --- | --- | --- |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| ____ | _________ | _____ | ___________________________ | ___________________________ | __ _____ |
| Figure 1 |  |  |  |  |  |

| DATE: 20XX0423 | MAINTENANCE REQUEST | DA FORM 5990-E |
| --- | --- | --- |

---------------------------------------------------------------- CUSTOMER DATA ----------------------------------------------------------------

| UIC: WBAKD0 | HHC, 13TH MP BN | PHONE: 765-9185 |
| --- | --- | --- |
| UTIL CODE: 0 |  |  |

----------------------------------------------------------------- ACTIVITY DATA -----------------------------------------------------------------

| SUP WON: | 581ST MAINTENANCE CO | PHONE: 734-7235 |
| --- | --- | --- |
| SUP UIC: WBBDXF |  | SHOP SEC: . |

--------------------------------------------------------------- EQUIPMENT DATA ---------------------------------------------------------------

| TYPE MNT REQ: 1 | ID: D NSN: 1005014123129 | MODEL: M240B . |
| --- | --- | --- |
| NOUN: Machine Gun 7.62mm | SER NUM: XXXXXXX | QTY: 00001 . |
| ORG WON : | PRIORITY: 05 . | FAILURE DETECTED: D . |
| MI/KM: ------------ | HOURS: ----- | ROUNDS: . |
| IN WARRANTY: | LEVEL OF WORK: F . | ADMIN NUM: DXX . |
| DEFICIENCY: Feed Tray Cracked |  |  |
| PD AUTHENTICATING SIGNATURE: ________________________________________________________ |  |  |

-------------------------------------------------------------- SIGNATURE DATA ------------------------------------------------------------------

| SUBMITTED BY: ______________________________ |  | ORD DATE: ___________ | MIL TIME: ___________ |
| --- | --- | --- | --- |
| ACCEPTED BY: ___________________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |

------------------------------------------------------------------ ACTION DATA ------------------------------------------------------------------

| WORK STARTED BY: _____________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |
| --- | --- | --- | --- |
| INSPECTED BY: ______________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |
| PICKED UP BY: _____________ | STATUS: __ | ORD DATE: ___________ | MIL TIME: ___________ |

--------------------------------------------------------------- COMPLETION DATA --------------------------------------------------------------

| QTY RPR: _____________ | QTY CONDEMNED: ___________ |  |  | NRTS: ___________ |  |
| --- | --- | --- | --- | --- | --- |
| EVAC WON: ______________________ |  |  | EVAC UNIT NAME: _______________________ |  |  |

Figure 2