BARTON COUNTY COMMUNITY COLLEGE

TROOP SCHOOL

FORT RILEY, KANSAS
SMALL ARMS
FINAL EXAM

January 10, 2006
Special Instruction:

This is an open book test.  You may use the reference publications shown below.  You may use notes taken during class.  DO NOT talk to other students at any time.  Speak ONLY to the Test Proctor.

If you have any questions, ask the Instructor NOW.
Select the answer you believe to be correct.  ONLY ONE correct answer is given for any question.

During this examination you will need:

A.  TM 9-1005-319-23&P

A. TM 9-1005-313-23&P
B. TM 9-1005-213-23&P

C. TM 9-1005-201-23&P
D. TM 9-1005-317-23&P

E. TM 9-1010-230-23&P
F.  AN AIMS ANSWER SHEET
G. A PENCIL OR PEN
H. CYCLE OF FUNCTION HANDOUT

I.  AR 700-138

J.  DA PAM 750-8
1. The firing pin on the M16A2 Rifle are replaced when defective at which level?

a. Operator

b. Organizational

c. General Support

d. Direct Support

2. Short recoil on the M16A2 Rifle could be caused by?

a. All of these

b. Damaged action spring

c. Missing bolt rings

d. Loose gas tube

3. The MK19 has ______ cycles of function?

a. 7

b. 8

c. 6

d. 9

4. Which lubricant is prohibited for use on the MK19?

a. LSA

b. CLP

c. GMD

d. LSAT

5. The corrective action for a bent accelerator stop on the Cal 50 is __________?

a. Replace

b. Straighten

c. Direct Support

d. General Support

6. If the Cal 50 machine gun fails to lock, one of the probably causes is _________?

a. Defective sear spring

b. Broken accelerator stop

c. Defective trigger bar

d. Defective extractor assembly

7. Services are scheduled ____________ as a minimum on the Cal 50 machine gun?

a. Annually

b. 60 days 

c. Semi-annually

d. Quarterly

8. When the safety fails to slide properly after cleaning and lubrication, the M249 machine gun should be __________?

a. Sent to direct support

b. Equipped with a new safety from stock

c. Deadlined until a new safety is ordered and received from direct support maintenance

d. Disassembled and all oil removed from the safety

9. The corrective action for a defective M249 MG sear is _________?

a. Organizational

b. General support

c. Direct support

d. Depot

10. The corrective maintenance level for damaged flanges on the M240 MG buffer assembly is _________?

a. Operator
b. Armorer
c. Depot
d. Direct support

11. When installing the M240 MG barrel assembly fewer than 2 clicks or more than ___clicks indicate defective parts?

a. 5

b. 7

c. 4

d. 8

12. When the M9 pistol fails to fire in double action, the probable cause is ________?

a. A defective release lever

b. A broken slide

c. A defective trigger bar spring

d. An obstruction in the receiver

13. Unit maintenance on the M9 pistol is limited to replacement of the pistol grips , minor hardware, and reversing the ________?

a. Magazine catch assembly

b. Firing pin

c. Hammer
d. Extractor

14. What publication governs The Army Maintenance Management System (TAMMS)?

a. AR 750-35

b. DA PAM 710-1

c. DA PAM 750-8
d. AR 700-138

15. Source code (XA) denotes?

a. Not authorized

b. Stocked item

c. Order next higher assembly

d. Non-recoverable

16. Which system has reportable small arms?

a. CFV

b. IFV

c. M88A1

d. M35A2

17. How long are the completed work orders (DA5990-E) maintained?

a. 160 days

b. 1 year

c. 90 days

d. 30 days

18. How long are the weapons issue/turn-in logs maintained?

a. 90 days

b. Until monthly inventory

c. 1 year

d. 6 months

19. What forms are used to issue other than individually assigned weapons?

a. DA 2062 & log

b. DA 3749 & DA 2062

c. Log only

d. DA 5513

20. How often should arms rooms be checked after duty hours?

a. Before and after midnight

b. 8 hours

c. 2 hours

d. 4 hours

21. The bolt and carrier assembly for the M16A2 rifle contains the firing pin, extractor, ejector and the ___________?

a. Cam pin

b. Hammer Spring

c. Pivot pin

d. Buffer

22. The part of the M16A2 rifle that provides firing control for the weapon and storage for basic cleaning materials is the __________?

a. Handle assembly and upper receiver

b. Lower receiver and buttstock assembly

c. Upper receiver and buttstock assembly

d. Upper and lower receiver assemblies

23. The bolt assembly for the M16A2 rifle should not be interchanged between weapons because to do so may __________?

a. Make the weapon serviceable

b. Jam the bolt assembly

c. Bind the receiver assembly

d. Result in injury or death

24. If the gas tube on the M16A2 in the upper receiver group hits the carrier key, or if the tube binds in the carrier key, the malfunctions should be corrected by adjusting the gas tube slightly in the area of the _________?

a. Gas tube

b. Handguards

c. Carrier

d. Upper receiver group

25. Armorer will request direct support to gage and conduct a 100% technical inspection on the M16A2 when?

a. Quarterly

b. Semi-annually

c. Annually

d. Monthly

26. The probable cause for a failure to fire on the M16A2 is?

a. Bent gas tube

b. Broken firing pin

c. Loose extractor

d. Weak recoil spring

27. The front sight post on the M16A2 is _____________?

a. Non-replaceable

b. Fixed

c. Only has rear sight

d. Adjustable

28. The selector level on the M16A2 should not be ________when closing the upper receiver?

a. On safe

b. On semi

c. On burst

d. Doesn’t matter

29. When the leading round is in line with the MK19 bolt face, describe the cycle of function.

a. Cocking

b. Extracting

c. Feeding

d. Charging

30. The action taken for a runaway gun on the MK19 is _____________?

a. Twist ammo belt

b. Charging Handle Down

c. Expend all remaining ammo

d. Put on safe

31. When removing sear housing from the MK19 receiver, the safety should be on ________?

a. Fire

b. Safe

c. Doesn’t matter

32. What component retracts the firing pin on the MK19?

a. Retractor assembly

b. Sear

c. Cocking assembly

d. Cocking Lever

33. The maintenance level for the MK19 Ogive plunger is __________?

a. Operator

b. Organizational

c. Direct Support

d. General Support

34. Probable cause of rounds not extracting on the MK19 would be __________?

a. Missing sear

b. Defective firing pin

c. Ejectors loose

d. Safety defective

35. Corrective action for bolt timing out of adjustment on the MK19 is _________?

a. (H)

b. (F)

c. (C)

d. (O)

36. What level of maintenance is responsible for lubricating the MK19 Ogive plunger?

a. (C)

b. (O)

c. (F)

d. (H)

37. The corrective action for a defective Cal 50 extractor assembly is _________?

a. General Support

b. Depot

c. Replace

d. Direct Support

38. When the Cal 50 machine gun fails to extract a cartridge, the probable cause is _____?

a. A defective sear

b. Incorrect timing

c. A defective cocking lever

d. Defective (pitted) chamber insufficient recoil

39. When the Cal 50 machine gun fails to fire, one of the probably causes is a defective ______?

a. Belt feed paw

b. Extractor switch

c. Sear

d. Bolt switch

40. If the Cal 50 machine gun fails to cock, the probable cause is a defective ______?

a. Sear

b. Firing pin extension assembly

c. Cocking lever pin

d. All of the above

41. Maintenance code “O” authorizes personnel to perform maintenance at the _______ level?

a. Operator/crew

b. Direct support

c. Organizational/unit

d. General support

42. The corrective action for a defective Cal 50 firing pin is ______?

a. Organizational

b. Direct support

c. General support

d. Depot

43. The fault status symbol on the DA5988-E for a defective Cal 50 sear would be ______?

a. /

b. X

c. –

d. 0

44. The M249 Machine Gun is __________?

a. Automatic

b. Gas assisted

c. Fired from open bolt

d. All of these

45. Organizational preventive maintenance checks and services (PMCS) should be performed every ________ days on the M249 MG?

a. 30

b. 60

c. 90

d. 120

46. When performing PMCS on the butt stock and shoulder assembly on the M249 MG, those items should be inspected for __________?

a. Cracks

b. Missing components

c. Shoulder piece locks in both positions

d. All of the above

47. Sluggish operation of the M249 machine gun would be caused by ______?

a. A bent cocking arm

b. Insufficient gas pressure

c. A bent feed tray

d. A dirty chamber

48. When the M249 machine gun fails to chamber a cartridge, the corrective action should be to check for _____________?

a. A dirty extractor

b. A broken ejector

c. A damaged spring

d. A broken sear

49. When the M249 machine gun fails to fire, the cause is probably ____________?

a. Carbon in gas inlets of regulator

b. Loose piston on operating rod

c. Damaged extractor

d. Damaged firing pin

50. The 1st two letter of the SMR code (XAOFF) denotes __________?

a. Direct support repairs item

b. Organizational orders item

c. Order next higher assembly

d. Not repairable at organizational level

51. The first step in troubleshooting a failure to fire on the M240 is _________?

a. Check for damaged driving spring rod

b. Check if ammunition is feeding

c. Check for damaged firing pin

d. Check for carbon build-up

52. Preventive maintenance checks and services (organizational) are performed ______ on the M240 machine gun?

a. Daily

b. Monthly

c. Quarterly

d. Weekly

53. The number setting for normal rate of fire on the gas regulator for the M240 is _____?

a. 1

b. 2

c. 3

d. 4

54. Look at figure 1, part number 3 is the _______? (M9 Pistol)

a. action spring

b. recoil spring

c. trigger spring

d. magazine spring

55.  Look at figure 1 again, part number 6 is the ________? (M9 Pistol)

a. magazine slide

b. bullet slide

c. safety slide

d. magazine spring follower

56. Look at figure 1 again, part number 7 is the ________? (M9 Pistol)

a. slide assembly

b. locking block

c. barrel

d. firing pin block

57. When ammunition fails to chamber in the M9 pistol, the first step in finding the cause is to check for ________?

a. broken recoil spring

b. dirt or an obstruction in the chamber 

c. damaged magazine spring

d. damaged feed ramp

58. How often are the arms room keys inventoried?

a. 30 days

b. 180 days

c. 90 days

d. Annually

59. How often is the security construction statement revalidated?

a. 5 years

b. Annually

c. 2 years

d. 3 years

60. The security checks (security screening) for unaccompanied access includes?

a. Medical records

b. Personnel files (201)

c. Local files

d. All the above

61. How often must the IDS be tested?

a. Weekly

b. Annually

c. Quarterly

d. Monthly

62. How is the operational load of ammo controlled?

a. FR 43-1

b. DA 2062

c. DA5513

d. DA 5515

63. How are the arms room keys controlled?

a. DA 2062

b. FR 43-1

c. DA 5513-R
d. DA 5515-R
64. How long may training ammo be temporarily stored in arms room?

a. 10 days

b. 14 days

c. 7 days

d. Not authorized

65. How long may individually assigned weapons be issued by DA 3749 alone?

a. 30 days

b. 1 week

c. 24 hours

d. 36 hours

 66. How long are monthly serial number inventories maintained (no discrepancies)?

a. 2 years

b. 4 years

c. 1 year

d. 3 years

67. May the unit Armorer conduct or assist on monthly inventories?

a. No

b. Yes 
68. When is a physical count inventory conducted?

a. Monthly

b. Daily

c. Change of custody of facility

d. New arms room officer

69. How much must arms racks/containers weigh so as not to be chained to the structure?

a. Doesn’t matter

b. 500 lbs.

c. 200 lbs.

d. 400 lbs.

70. What items must be posted on the exterior of the arms room entrance?

a. Access roster

b. Restricted warning

c. Alarm protection

d. Both b and c

71. How long are the physical count inventories maintained?

a. 2 weeks

b. Until monthly inventory

c. 90 days

d. 1 year

72. What are the lighting requirements for arms rooms?

a. None

b. Light switch not accessible

c. Both b & d

d. Vandal resistant guard on light

73. What type locks are required for arms rooms?

a. Brass

b. High security

c. Series 200/5200

d. Both b & c

 74. Is a master authorization list required?    
a. Yes

b. No

75. May privately owned weapons be stored in the same rack or container with military weapons?

a. Yes

b. No

76. How many keys come with the high security padlock?

a. 2

b. 3

c. 1

d. 4

77.Whom may conduct monthly serial number weapons inventories?

a. Officers

b. Warrant officers

c. All these

d. NCOs

78. What reference authorizes storage of night vision devices in arms rooms on Ft. Riley?

a. AR 190-11

b. FR 190-1

c. CG policy letter 12-1(Riley’s Rules)
d. DA Pam 710-2-1

 79. Responsibility for registration of privately owned firearms is?

a. Armorer

b. Commander

c. Arms room officer

d. PMO

80. What is the Julian date for August 20th (not leap year)?
a. 233

b. 231

c. 212

d. 232
81. The 4th position of the SMR code determines who may ________?

a. Install

b. Dispose of

c. Repair completely

d. Requisition

82. When the organizational TM does not have an NMC column, the armorer determines the status using?

a. Operator’s manual

b. Proper function

c. Safety

d. All of the above

83. What publication governs reportable weapons?

a. DA PAM 738-750

b. AR 750-1

c. AR 700-138

d. AR 138-700

84. Maintenance code (F) is what level of maintenance?

a. Organizational 

b. Depot

c. Intermediate

d. General support

85. Recoverability code (A) denotes?

a. Non-recoverable

b. Organizational only

c. Direct support repair

d. Special handling

86. What DA form is used to request direct support maintenance repair (work order)?

a. DA 2404 

b. DA5990-E

c. DA 5988-E

d. Both a & c

 87. The fault status will be listed in what column of the DA 5988-E

a. 1

b. 3

c. 2

d. 4

88. The DA form 5988-E for completed services is kept on file how long?

a. Until next service

b. 180 days

c. 90 days only

d. 1 year

89. Two (02) priority requisitions are determined by the?

a. Commander

b. TM stated NMC

c. Armorer

d. Executive officer

90. The status symbols used on the DA 5988-E include?

a. X

b. Circle X

c. /

d. All of these

91. Who may authorize circle X on NMC weapons?

a. Armorer

b. Commander

c. Arms room officer

d. Supply sergeant

92. What is the SMR code for the charging handle for the M16A2?

a. PAFFZ

b. PAOZZ

c. PADDA

d. XAOZZ
93. How often are a 100% technical inspection and gauging, required on all small                                        
arms?
a. Monthly

b. Quarterly

c. Annually

d. Biannually

94. All small arms are inspected _______ by the armorer?

a. Annually

b. Monthly

c. Quarterly

d. Daily

95. Operator maintenance is the most important level of maintenance?

a. True

b. False

96. The status symbol X is used on DA 5988-E when?

a. TM states NMC

b. MWO not applied

c. Weapon is dirty

d. Operator not assigned

97. What level of maintenance replaces the firing pin on the M16A2?

a. Operator

b. Direct support

c. Armorer

d. General support

98. What level of maintenance replaces the cable on the M10 charger on the M48, Cal 50 machine gun?

a. F

b. D

c. O

d. H
 99. What does the last part of a TM (24P) represent?

a. Organizational parts manual

b. Direct support parts manual

c. General support parts manual 

d. All of these

100. The M1 series tank has ________ reportable small arms?

a. 3

b. 2

c. 1

d. None


- 12 -

