BARTON COUNTY COMMUNITY COLLEGE

TROOP SCHOOL

FORT RILEY, KANSAS

                                            SMALL ARMS
FINAL EXAM I
14 December 2012

(All Previous Versions are rescinded)
14 December 2012
Special Instruction:

This is an open book test. You may use the reference publications shown below. You may use notes taken during class. DO NOT talk to other students at any time. Speak ONLY to the Test Proctor.

If you have any questions, ask the Instructor NOW.
Select the answer you believe to be correct. ONLY ONE correct answer is given for any question.

During this examination you will need:

A.  TM 9-1005-319-23&P
B.  TM 9-1005-313-23&P
C.  TM 9-1005-213-23&P

D.  TM 9-1005-201-23&P
E.   TM 9-1005-317-23&P

F.   TM 9-1010-230-23&P
G.  AN ANSWER SHEET
H.  A PENCIL OR PEN
I.   CYCLE OF FUNCTION HANDOUT

J.   AR 700-138

K.  DA PAM 750-8
L.  ANY NOTES
1. The firing pin on the M16A2 Rifle is replaced when defective at which level?

a. Operator

b. Contractor
c. General Support

d. Field Maintenance
2. Short recoil on the M16A2 Rifle could be caused by?

a. All of these

b. Damaged action spring

c. Missing bolt rings

d. Loose gas tube

3.The MK19 has ______ cycles of function?

a. 7

b. 8

c. 6

d. 9

4. Which lubricant is prohibited for use on the MK19?

a. LSA

b. CLP

c. GMD

d. LSAT

5. The corrective action for a bent accelerator stop lock on the Cal 50 is __________?

a. Replace

b. Straighten

c. Field Maintenance
d. General Support

6. If the Cal 50 machine gun fails to chamber, one of the probably causes is _________?

a. Defective sear spring

b. Broken accelerator stop

c. Defective trigger bar

d. Defective extractor assembly
7.  Services are scheduled __________ as a minimum on the Cal 50 machine gun?

a. Annually

b. 60 days 

c. Semi-annually

d. Quarterly

8.  When the safety fails to slide properly after cleaning and lubrication, the M249 MG should be __________?

a. Sent to field maintenance
b. Equipped with a new safety from stock

c. Deadlined until a new safety is ordered and received from direct support maintenance

d. Disassembled and all oil removed from the safety
9.  The corrective action for a defective M249 MG sear is _________?

a. Contractor
b. General support

c. Field Maintenance
d. Depot

10.  The corrective maintenance level for damaged flanges on the M240 MG buffer assembly is _________?

a. Operator
b. Armorer
c. Depot
d. Field Maintenance
11.  Failure to feed on the M240 MG could be caused by ___?

a. Defective ejector
b. Damaged feed tray
c. Broken extractor
d. Defective firing pin

12.  When the M9 pistol fails to fire in double action, the probable cause is ________?

a. A defective release lever         c.  A defective trigger bar               

                                                             spring           
b. A broken slide                           d.  An obstruction in the                                                                                           
                                                                           receiver
13.  Unit maintenance on the M9 pistol is limited to replacement of the pistol grips, minor hardware, and reversing the ________?

a. Magazine catch assembly

b. Firing pin

c. Hammer
d. Extractor

14.  What publication governs The Army Maintenance Management System (TAMMS)?

a. AR 750-35

b. DA PAM 710-1

c. DA PAM 750-8
d. AR 700-138

15.  Source code (XA) denotes?

a. Not authorized

b. Stocked item

c. Order next higher assembly

d. Non-recoverable

16.  The armor will request direct support to Gage and conduct a 100% technical inspection ___________?

a. Quarterly
b. Semi-annually
c. Annually
d. Monthly
17.  The probable cause for a failure to fire (M16A2) is?
a. Bent gas tube            c. Loose extractor
b. Broken firing pin        d. Weak recoil spring
18. The action taken for a runaway gun on the MK19 is _____________?

a. Twist ammo belt

b. Charging Handle Down

c. Expend all remaining ammo

d. Put on safe

19. When removing sear housing from the MK19 receiver, the safety should be on ________?

a. Fire

b. Safe

c. Doesn’t matter

20. What component retracts the firing pin on the MK19?

a. Retractor assembly

b. Sear

c. Cocking assembly

d. Cocking Lever

21. The maintenance level for the MK19 Ogive plunger is __________?

a. Operator

b. Armorer
c. Field Maintenance
d. General Support

22. Probable cause of rounds not extracting on the MK19 would be __________?

a. Missing sear

b. Defective firing pin

c. Ejectors loose

d. Safety defective
23. When the Cal 50 machine gun fails to eject a cartridge, the probable cause is _____?

         a. Defective sear                   c. Defective extractor
b. Defective cocking lever    d. Defective ejector                                   
24. When the Cal 50 machine gun fails to fire, one of the probably causes is a defective ______?

a. Belt feed paw

b. Extractor switch

c. Sear
d. Bolt switch
25. If the Cal 50 machine gun fails to cock, the probable cause is a defective ______?
a. Sear

b. Firing pin extension assembly

c. Cocking lever pin

d. All of the above

26. Maintenance “Code” F authorizes personnel to perform maintenance at the _______ level?

a. Operator/crew

b. Field Maintenance
c. Organizational/unit

d. General support

27. The corrective action for a defective Cal 50 firing pin is ______?

a. Organizational

b. Field Maintenance
c. General support

d. Depot

28. The fault status symbol on the DA5988-E for a defective Cal 50 sear would be ______?

a. /
b. X

c. –

d. 0

29. The M249 Machine Gun is __________?

a. Automatic                  c. Fired from open bolt       
b. Gas assisted              d. All of these
30. Organizational preventive maintenance checks and services (PMCS) should be performed every ________ days on the M249 MG?

a. 30

b. 60

c. 90

d. 120

31. When performing PMCS on the butt stock and shoulder assembly on the M249 MG, those items should be inspected for __________?

a. Cracks

b. Missing components

c. Shoulder piece locks in both positions

d. All of the above

32. Sluggish operation of the M249 machine gun would be caused by ______?

a. A bent cocking arm

b. Insufficient gas pressure

c. A bent feed tray

d. A dirty chamber

33. When the M249 machine gun fails to chamber a cartridge, the corrective action should be to check  for _____________?

a. A dirty extractor

b. A broken ejector

c. A damaged spring

d. A broken sear
34. When the M249 machine gun fails to fire, the cause is probably ____________?

a. Carbon in gas inlets of regulator

b. Loose piston on operating rod

c. Damaged extractor

d. Damaged firing pin

35. The first step in troubleshooting a failure to fire on the M240 MG is _________?

a. Check for damaged driving spring rod

b. Check if ammunition is feeding

c. Check for damaged firing pin

d. Check for carbon build-up

36. Organizational preventive maintenance checks and services are performed ______ on the M240 machine gun?

a. Daily

b. Monthly

c. Quarterly

d. Weekly

37.  How long are the completed work orders (DA5990-E) maintained _____?

a. 160 days
b. 1 year
c. 90 days
d. 180 days
38. What DA form is used to request support maintenance?
a. DA 2404
b. DA 2407-E

c. DA 5988-E

d. DA 5513-E

39.   The status symbol X is used when? 

a. TM stated NMC
b. Urgent MWO not applied
c. Safety

d. All the above

40.   How long are the DA 5988-E (completed services) maintained?

a. 90 days       c. Until next service
b. 180 days     d. 1 year

- 4 -

