BARTON COUNTY COMMUNITY COLLEGE

TROOP SCHOOL

FORT RILEY, KANSAS

SMALL ARMS
PRETEST
ARMORER AND SUPERVISOR

14 DECEMBER 2012

(All Previous Versions are rescinded)
 14 December 2012
Special Instruction:

Armorer Course students complete Questions 1 –  20.

Armorer Supervisor Course complete Questions 1 – 10.
This is an open book test.  You may use the reference publications shown below.   DO NOT talk to other students at any time.  Speak ONLY to the Test Proctor.

If you have any questions, ask the Instructor NOW.
Select the answer you believe to be correct.  ONLY ONE correct answer is given for any question.

During this examination you will need:

A.  TM 9-1005-319-23&P
B.  TM 9-1005-313-23&P
C.  TM 9-1005-213-23&P

D.  TM 9-1005-201-23&P
E.   TM 9-1005-317-23&P

F.   TM 9-1010-230-23&P
G.  AN ANSWER SHEET
H.  A PENCIL OR PEN
I.   CYCLE OF FUNCTION HANDOUT

J.   AR 700-138

K.  DA PAM 750-8
1.  The firing pin on the M16A2 Rifle is replaced when   
defective at which level?

a. Operator

b. Armorer
c. General Support

d. Field Maintenance
 2.  Short recoil on the M16A2 Rifle could be caused by?

a. All of these

b. Damaged action spring

c. Missing bolt rings

d. Loose gas tube

  3. The MK19 has ______ cycles of function?

a. 7        c. 6
b. 8        d. 9
   4.  Which lubricant is prohibited for use on the MK19?

a. LSA

b. CLP

c. GMD

d. LSAT

5.  The corrective action for a damaged firing pin extension                                      .    on the Cal 50 is __________?

a. Replace

b. Straighten

c. Field Maintenance
d. General Support

   6.  If the Cal 50 machine gun fails to chamber, one of the 
probably causes is _________?

a. Defective sear spring

b. Broken accelerator stop

c. Defective trigger bar

d. Defective extractor assembly

    7.   Services are scheduled _______ as a minimum on the Cal 
50 machine gun?

a. Annually

b. 60 days 

c. Semi-annually

d. Quarterly
        8.   When the safety fails to slide properly after cleaning and 

        lubrication, the M249MG should be________?

a. Sent to direct support

b. Equipped with a new safety from stock

c. Deadlined until a new safety is ordered and received from direct support maintenance

d. Disassembled and all oil removed from the safety
 9.   The corrective action for a defective M249 MG  sear is______?

a. Organizational

b. General support

c. Direct support

d. Depot

 10.  The corrective maintenance level for damaged flanges on the M240 MG buffer assembly is _________?

a. Operator
b. Armorer
c. Depot
d. Field Maintenance
 11.  Failure to feed on the M240 MG could be caused by ____?

a. Defective ejector
b. Damaged feed tray
c. Broken extractor
d. Defective firing pin

   12.  When the M9 pistol fails to fire in double action, the 
probable cause is ________?

a.  A defective release lever        c.  A defective  trigger                                                                   
                                                           bar spring 
 b.  A broken slide                         d. An obstruction in                                                                   





         the receiver  
    13. Unit maintenance on the M9 pistol is limited to          
replacement of the pistol grips , minor hardware, and 
reversing the ________?


 a.  Magazine catch assembly


 b. Firing pin

     c.  Hammer
          d. Extractor
          14.  What publication governs The Army Maintenance 

          Management System (TAMMS)?

   a.  AR 750-35

   b.  DA PAM 710-1

   c.  DA PAM 750-8
        d.  AR 700-138
15.  Source code (XA) denotes?

    a.  Not authorized

    b.  Stocked item

        c.  Order next higher assembly

    d.  Non-recoverable
      16. The armor will request direct support to Gage and 
conduct a 100% technical inspection ___________?

     a.  Quarterly
     b.  Semi-annually
     c.  Annually
     d.  Monthly
             17.  The probable cause for a failure to fire (M16A2) is?

       a.  Bent gas tube
       b.  Broken firing pin
   c.  Loose extractor
   d.  Weak recoil spring
            18.  The action taken for a runaway gun on the MK19 is 

        _____________?

   a.  Twist ammo belt

    b.  Charging Handle Down

    c.  Expend all remaining ammo
    d.  Put on safe

             19.  When removing sear housing from the MK19 receiver, 
       the safety should be on ________?

  a.  Fire

  b.  Safe

  c.  Doesn’t matter

              20.  What component retracts the firing pin on the 

  

MK19?

  a.  Retractor assembly

  b.  Sear

  c.  Cocking assembly

       d.  Cocking Lever


- 1 -

