EVEREADY BATTERY CO INC -- CARBON-ZINC BATTERY -- 6135-00-985-7845 

=====================  Product Identification  =====================

Product ID:CARBON-ZINC BATTERY

MSDS Date:08/05/1994

FSC:6135

NIIN:00-985-7845

MSDS Number: BWNLG

=== Responsible Party ===

Company Name:EVEREADY BATTERY CO INC

Address:CHECKERBOARD SQ

City:ST LOUIS

State:MO

ZIP:63164

Country:US

Info Phone Num:314-982-2000/216-835-7368

Emergency Phone Num:314-982-2000

CAGE:83740

=== Contractor Identification ===

Company Name:EVEREADY BATTERY CO INC

Address:CHECKERBOARD SQ

Box:City:ST LOUIS

State:MO

ZIP:63164

Country:US

Phone:216-835-7368

CAGE:83740

=============  Composition/Information on Ingredients  =============

Ingred Name:CARBON BLACK

CAS:1333-86-4

RTECS #:FF5800000

Fraction by Wt: 3-7% %

Other REC Limits:NONE RECOMMENDED

OSHA PEL:3.5 MG/M3

ACGIH TLV:3.5 MG/M3; 9394

Ingred Name:MANGANESE DIOXIDE

CAS:1313-13-9

RTECS #:OP0350000

Fraction by Wt: 15-31% %

Other REC Limits:NONE RECOMMENDED

OSHA PEL:5 MG MN/M3 CEILING

ACGIH TLV:5 MG MN/M3

Ingred Name:ZINC  (SARA III)

CAS:7440-66-6

RTECS #:ZG8600000

Fraction by Wt: 7-42% %

Other REC Limits:NONE RECOMMENDED

EPA Rpt Qty:1000 LBS

DOT Rpt Qty:1000 LBS

Ingred Name:ZINC CHLORIDE  (SARA III)

CAS:7646-85-7

RTECS #:ZH1400000

Fraction by Wt: 2-7% %

Other REC Limits:NONE RECOMMENDED

OSHA PEL:1 MG/M3

ACGIH TLV:1MG/M3/2STEL(FUME)94

EPA Rpt Qty:1000 LBS

DOT Rpt Qty:1000 LBS

=====================  Hazards Identification  =====================

LD50 LC50 Mixture:NOT GIVEN FOR PRODUCT AS A WHOLE

Routes of Entry: Inhalation:NO
Skin:NO  Ingestion:NO

Reports of Carcinogenicity:NTP:NO    IARC:NO
OSHA:NO

Health Hazards Acute and Chronic:SWALLOWING A BATTERY CAN BE HARMFUL.

    CONTENTS OF AN OPEN BATTERY CAN CAUSE SERIOUS CHEMICAL BURNS OF THE

    MOUTH, ESOPHAGUS, GI TRACT, SKIN, EYES AND/OR RESPIRATORY TRACT.

    CAUSE GI DISTURBANCES.

Explanation of Carcinogenicity:THIS COMPOUND CONTAINS NO INGREDIENTS AT

    CONCENTRATIONS OF 0.1% OR GREATER THAT ARE CARCINOGENS OR SUSPECT

    CARCINOGENS.

Effects of Overexposure:EYE IRRITATION, SKIN IRRITATION, UPPER

    RESPIRATORY TRACT IRRITATION, BURNS OF THE MOUTH, ESOPHAGUS, GI

    TRACT, SKIN, EYES AND/OR RESPIRATORY TRACT.

Medical Cond Aggravated by Exposure:NONE SPECIFIED BY MANUFACTURER.

=======================  First Aid Measures  =======================

First Aid:EYES: FLUSH WITH WATER FOR 15 MINUTES WHILE HOLDING EYELIDS

    OPEN. GET MEDICAL ATTENTION. SKIN: REMOVE CONTAMINATED CLOTHING.

    WASH WITH SOAP AND WATER. IF IRRITATION PERSISTS, GET MEDICAL

    ATTENTION. IN HALATION: REMOVE TO FRESH AIR. RESTORE BREATHING. GET

    MEDICAL ATTENTION. INGESTION: DO NOT INDUCE VOMITING. GET MEDICAL

    ATTENTION.CALL NATIONAL BATTERY INGESTION HOTLINE COLLECT

    (202-625-3333).

=====================  Fire Fighting Measures  =====================

Flash Point:NONFLAMMABLE

Lower Limits:NOT GIVEN

Upper Limits:NOT GIVEN

Extinguishing Media:ANY CLASS OF EXTINGUISHING MEDIUM IS APPROPRIATE

Fire Fighting Procedures:FIRE FIGHTERS SHOULD USE NIOSH APPROVED SCBA &

    FULL PROTECTIVE EQUIPMENT WHEN FIGHTING CHEMICAL FIRE. USE WATER

    SPRAY TO COOL NEARBY CONTAINERS EXPOSED TO FIRE.

Unusual Fire/Explosion Hazard:DO NOT USE DIRECT STREAM OF WATER ON

    FIRE. TOXIC GASES ARE RELEASED DURING COMBUSTION.

==================  Accidental Release Measures  ==================

Spill Release Procedures:COLLECT FOR DISPOSAL. IF BATTERIES HAVE

    RUPTURED, USE PROPER PERSONAL PROTECTIVE EQUIPMENT.

Neutralizing Agent:NONE SPECIFIED BY MANUFACTURER.

======================
Handling and Storage  ======================

Handling and Storage Precautions:STORE IN A COOL, WELL VENTILATED AREA.

    ELEVATED TEMPERATURES CAN RESULT IN SHORTENED BATTERY LIFE.

Other Precautions:IF POTTING OR SEALING THE BATTERY IN AN AIRTIGHT OR

    WATERTIGHT CONTAINER IS REQUIRED, CONSULT YOUR EVEREADY BATTERY

    COMPANY FOR PRECAUTIONARY SUGGESTIONS.

=============  Exposure Controls/Personal Protection  =============

Respiratory Protection:NOT NECESSARY UNDER NORMAL CONDITIONS.

Ventilation:NOT NECESSARY UNDER NORMAL CONDITIONS.

Protective Gloves:NEOPRENE OR NATURAL RUBBER IF LEAKING.

Eye Protection:SAFETY GLASSES OR GOGGLES IF LEAKING.

Other Protective Equipment:AN EMERGENCY EYE WASH STATION AND SHOWER

    SHOULD BE AVAILABLE.

Work Hygienic Practices:WASH WITH SOAP AND WATER AFTER HANDLING PRODUCT

    AND BEFORE EATING DRINKING OR SMOKING.

Supplemental Safety and Health

THIS ITEM IS CONSIDERED BY MANUFACTURER TO BE AN ARTICLE. DO NOT

    DISPOSE IN FIRE, RECHARGE, PUT IN BACKWARDS, MIX WITH USED OR OTHER

    BATTERY TYPES -- MAY EXPLODE OR LEAK AND CAUSE PERSONAL INJURY. KEE

    P AWAY FROM SMALL CHILDREN.

==================  Physical/Chemical Properties  ==================

HCC:N1

Boiling Pt:B.P. Text:NOT GIVEN

Melt/Freeze Pt:M.P/F.P Text:NOT GIVEN

Vapor Pres:NOT GIVEN

Vapor Density:NOT GIVEN

Spec Gravity:NOT GIVEN

Evaporation Rate & Reference:NOT GIVEN

Solubility in Water:INSOLUBLE

Appearance and Odor:SEALED CELL

=================  Stability and Reactivity Data  =================

Stability Indicator/Materials to Avoid:YES

NONE SPECIFIED BY MANUFACTURER.

Stability Condition to Avoid:EXTREME TEMPERATURES

Hazardous Decomposition Products:NONE SPECIFIED BY MANUFACTURER.

====================  Disposal Considerations  ====================

Waste Disposal Methods:PREVENT WASTE FROM CONTAMINATING SURROUNDING

    ENVIRONMENT. DISCARD ANY PRODUCT, RESIDUE, DISPOSAL CONTAINER OR

    LINER IN ACCORDANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS.

 Disclaimer (provided with this information by the compiling agencies):

 This information is formulated for use by elements of the Department

 of Defense.  The United States of America in no manner whatsoever,

 expressly or implied, warrants this information to be accurate and

 disclaims all liability for its use.  Any person utilizing this

 document should seek competent professional advice to verify and

 assume responsibility for the suitability of this information to their

 particular situation.

