M16/M4 WRITTEN PE (JULY 2012)

1.	The firing of two (2) rounds on a single pull of the trigger with selector lever set on semi is due to?

a. A bent magazine spring

b. A short recoil spring

c. The gas port was plugged

d. The weapon failed an organizational level test as described in TM 9-1005-319-23&P	(Pg 0005-18)

2.	A failure to cock in the M16A2 rifle is most likely to occur because?

a. The firing pin has broken

b. The magazine catch is loose

c. The sear and/or trigger (firing mechanism) are defective					(Pg 0005-11)

d. The magazine spring is weak

3.	Which statement is correct concerning the M16A2 rifle?

a. It is a gas piston operated weapon

b. It is a combination fixed/blowback operated weapon

c. It is an air cooled and recoil operated weapon			(Pg 0002-1)(In PE/COF binder)

d. It is a gas operated, air cooled, lightweight, magazine fed and semi-automatic or burst weapon

4.	Which of the following parts are not interchangeable between weapons?

a. Bolt assemblies									(Pg 0012-6/b)

b. Firing pin retaining pin

c. Handguards

d. The extractor

5.	The most probable cause in a failure to extract would be?

a. Broken trigger

b. Defective action spring

c. Defective extractor spring assembly						(Pg 0005-8)

d. Defective ejector spring

6.	A bent gas tube in the receiver would cause a failure to?

a. Cock		c. Lock									(Pg 0005-5)

b. Extract	d. Eject

7.	The operator states his weapon does not chamber, therefore the armorer must troubleshoot the?

a. Parts in step of feeding steps 1 through 8

b. The parts of the firing mechanism

c. The action spring									(Pg 0005-3)

d. The ejector spring

8.	Which one of the following malfunctions could be caused by short recoil?

a. Failure to cock

b. Failure to extract

c. Failure to eject

d. All of the above							(Pg 0005-8 through 11)

9.	Which part listed below, if not installed, will cause injury or death to the operator? (While firing the first round)

a. The automatic sear

b. The bolt carrier key

c. The firing pin

d. Bolt cam pin							(Pg 0011-7/b)

10.	In extreme arctic conditions, the armorer would order which of the following lubricants for the M16A2?

a. GP oil

b. CLP

c. LSA

d. LAW									(Pg 0008-27)

11.	The first step in troubleshooting a failure to lock is?

a. Loose bolt carrier key

b. Short recoil

c. Check for bent gas tube

d. Check for missing bolt cam pin					(Pg 0005-4)

12.	PMCS (organizational level) for the M16A2 rifle is scheduled?

a. Quarterly								(Pg 0008-2)

b. Monthly

c. Annually

d. Daily

13.	The third position of the SMR code determines?

a. How you get a part

b. Who can install, replace or use the item				(Pg 0038-2)

c. Who determines disposition

d. Who can do complete repair on the item

14.	Which part listed below could cause a loss of gas pressure?

a. Extractor pin

b. Bolt carrier key

c. Helical spring

d. Worn bolt rings							(Pg 0012-7)

15.	When parts of the upper receiver need replacement on the M16A2, the armorer using the SMR code may replace the?

a. Flash suppressor	b. Sear		c. None of these	d. Gas tube	(Pg 0039-17/35)

16.	The firing pin is replaced by?

a. (H)		b. (D)		c. (F)		d. (O)				(Pg 0039-9)

17.	The forward assist assembly is replaced by?

a. (O)		b. (D)		c. (F)		d. (H)				(Pg 0039-29)

18.	Quarterly PMCS on the M16A2 rifle is best described?

a. Finding and correcting small faults before they become major maintenance problems	(Pg 0007-1)

b. Recording faults and informing the operator

c. Do PMCS so the 1SG can take action on cleaning

d. Primary maintenance concerning shortcomings

19.	Disassembly of the lower receiver firing mechanism is not authorized at organizational level because?

a. The special tools needed are too costly to have in each arms room

b. Repeated removal of the pins will enlarge the holes and result in malfunctions and loss of parts

c. Incorrect re-assembly would cause a failure to unlock

d. It is very hard for the unit armorer to learn

20.	The hammer is cocked by the?

a. Bolt carrier								(In PE/COF binder)

b. Locking lugs making contact with the barrel

c. By the expansion of the action spring

d. Pressure exerted by the bolt locking lugs

21.	Broken or missing parts of the firing mechanism can cause a failure in what two steps of the cycle of function? Firing and Cocking					(Pg 0005-6/10)

22.	What component provides the source of power to move the bolt carrier group forward after firing? 	Action Spring							(In PE/COF binder)

23.	A broken magazine catch could cause a failure in which?

a. Cock		b. Feed		c. Chamber		d. Fire		(Pg 0005-2)

24.	What is the NSN for the M16A2 barrel assembly? 1005-01-146-7684	(Pg 0039-17)

25.	What malfunction could occur if the bolt rings are not staggered? 	Short Recoil	(Pg 0005-12)