M240 SERIES MACHINE GUN WRITTEN PE (JULY 2012)

1.	What is the corrective action for a buffer assembly latch damaged?

a. Organization	b. Operator	c. Direct Support	d. Depot		(Pg 0035 00-3)

2.	The M240 series Machine Gun employs ___________ operation?

a. Recoil	b. Gas		c. Blowback		d. Manually			(Pg 0003 00-1)

3.	The M240C Machine Gun is belt fed from the _________ side?

a. Right						c. It is manually fed		(In PE/COF binder)

b. Is capable of both right and left hand feed	d. Left				(Pg 0002 00-5)

4.	The corrective action for a broken heat shield on the M240B is?		(Pg 0032 00-3)

a. Organizational	b. Direct Support	c. General Support	d. None of the above

5.	The setting for normal rate of fire on the M240 is?				(Pg 0008 00-10)

a. One		b. Two		c. Three	d. All the above			(In PE/COF binder)

6.	Preventive maintenance checks and services are performed?

a. Weekly	b. Monthly	c. Quarterly	d. Daily				(Pg 0005 00-1)

7.	When installing the barrel assembly into the receiver, less than two or more than seven clicks indicates?

a. Defective parts				c. That the receiver is serviceable	(Pg 0005 00-11)

b. That the barrel assembly is serviceable	d. That the barrel release is serviceable

8.	Replacement of the sear is accomplished at __________ level?

a. Organizational	b. General Support	c. Operator	d. Direct Support	(Pg 0040 00-3)

9.	The M240 gas regulator plug (old style) is designed with _____ inlet settings to maintain the rate of fire?

a. Three	b. Two		c. Four		d. One					(Pg 0008 00-10)

10.	Of the following assemblies, which one is not replaceable at Direct Support level?

a. Trigger housing assembly			c. Bolt assembly

b. Cover assembly				d. Receiver assembly			(Pg 0031 00-3)

11.	A defective safety is corrected by?

a. (O)		b. (F)		c. (H)		d. (D)					(Pg 0040 00-3)

12.	The firing pin is replaced by?

a. (F)		b. (O)		c. (C)		d. (H)					(Pg 0037 00-1)

13.	The NSN for the ejector removing tool is? 4933-01-038-7179		(Pg 0051 00-3)

14.	A defective extractor is replaced by?						(Pg 0038 00-1)

a. Operator		b. Organizational	c. Direct Support	d. General Support

15.	The corrective action for an M240B defective feed tray hinge pin is?

a. Organizational	b. Direct Support	c. General Support	d. Operator	(Pg 0031 00-3)

16.	The M240 series Machine Gun fires from the ________ bolt position?

a. Closed	b. Centered	c. Open	d. None of the above			(Pg 0003 00-1)

17.	During what step in the cycle of operation does the ejector compress into the face of the bolt?

a. Locking	b. Ejecting	c. Feeding	d. Chambering				(In PE/COF binder)

18.	Which parts are replaced by the armorer?					(Pg 0038 00-1)

a. Carrying handle	b. Trigger spring	c. Extractor assembly		d. Gas piston

19.	What is the source of power to move the bolt and operating rod assembly forward during chambering?

a. Gas pressure		c. Driving spring rod					(In PE/COF binder)

b. Recoil of the bolt		d. All the above

20.	What component holds the bolt and operating rod assembly to the rear in a cocked position?

a. Sear		b. Buffer	c. Gas pressure	d. Trigger pin			(In PE/COF binder)

21.	Which of the following would cause a failure to feed?

a. Defective extractor			c. Defective cover assembly			(Pg 0004 00-4)

b. Defective barrel release		d. Defective ejector spring

22.	Which of the following would cause a malfunction?

a. Defective gas regulator			c. Insufficient gas pressure

b. Obstruction in the gas pressure system	d. All the above			(Pg 0004 00-5)

23.	Which of the following would NOT cause a failure to cock?			(Pg 0004 00-6)

a. Broken trigger spring		c. Broken sear spring

b. Worn firing pin			d. Worn sear notch on the bolt and operating rod

24.	Which of the following would cause a failure to eject?			(Pg 0004 00-5)

a. Broken extractor spring			c. Carbon accumulation on the firing pin aperture

b. Ejector frozen into the face of the bolt	d. None of the above

25.	What would be the corrective action for a broken feed lever in the cover assembly?

a. Have the crew replace it			c. Evacuate to Direct Support		(Pg 0042 00-3)

b. Evacuate to General Support		d. None of the above