MK19 WRITTEN PE (JULY 2012)

1.	The MK19 is a _____________ operated weapon?

a. Recoil	b. Blowback	c. Manual	d. Gas		(In PE/COF binder) (Pg 0002 00-1)

2.	The effective range of the MK19 is?			(In PE/COF binder) (Pg 0002 00-5)

a. 1200 meters	b. 1000 meters	c. 1500 meters	d. 2200 meters

3.	The MK19 has ________ cycles of function?

a. 7		b. 8		c. 9		d. 6		(In PE/COF binder) (Pg 0003 00-1)

4.	When the leading round is in line with the bolt face, describe the cycle of function?

a. Cocking	b. Charging	c. Feeding	d. Extracting	(In PE/COF binder) (Pg 0003 00-1)

5.	The action taken for a runaway gun is?

a. Twist the ammo belt		c. Charging handles down	(In PE/COF binder) (Pg 0003 00-5)

b. Expend all remaining ammo	d. Put on safe

6.	When removing the sear housing from the receiver, the safety should be on?

a. Fire		b. Safe		c. Doesn’t matter	d. Don’t know		(Pg 0048 00-2)

7.	The MK19 is fired with the ____________?

a. Bolt to the rear	b. Bolt forward	c. Bolt half-cocked	(In PE/COF binder) (Pg 0003 00-3)

8.	Which lubricant is prohibited for use on the MK19?

a. LSA		b. GMD	c. CLP		d. LSAT			(Pg 0081 00-1)

9.	What component retracts the firing pin on the MK19?

a. Retractor assembly		c. Cocking assembly

b. Cocking lever		d. Sear					(In PE/COF binder) (Pg 0003 00-2)

10.	How many inches of steel will the round penetrate?

a. 1		b. 3		c. 2		d. 4			(In PE/COF binder) (Pg 0091 00-2)

11.	The maintenance level for the Ogive plunger is?

a. Operator	b. Organizational	c. Direct Support	d. General Support	(Pg 0046 00-1)

12.	The cover hinge pins are replaced by?

a. (H)		b. (F)		c. (O)		d. (D)			(Pg 0097 00-3)

13.	The vertical cam assembly is replaced by?

a. Direct Support	b. Organizational	c. Operator	d. Depot	(Pg 0097 00-3)

14.	Replacement of the top cover is?

a. Operator	b. Direct Support	c. Organizational	d. General Support	(Pg 0097 00-3)

15.	The internal lubrication of the sear assembly is performed by?

a. (F)		b. (D)		c. (O)		d. (H)				(Pg 0072 00-10)

16.	The first step in troubleshooting sluggish firing is?

a. Bolt face dry		c. Recoil springs weak

b. Bolt rails burred		d. Carbon buildup in chamber		(Pg 0013 00-1)

17.	Jammed bolt during charging could be caused by?

a. Vertical cam bent		c. Extractor spring defective			(Pg 0005 00-5)

b. Cover hinge pin missing	d. Broken sear

18.	Probable cause of rounds not extracting would be?

a. Missing sear			c. Ejectors loose

b. Defective firing pin		d. Safety defective				(Pg 0010 00-2)

19.	Feed slide adjustment with the adjustable secondary drive lever is performed by?

a. (H)		b. (O)		c. (C)		d. (F)				(Pg 0042 00-1)

20.	What level of maintenance is responsible for lubricating the Ogive plunger?

a. (C)		b. (F)		c. (O)		d. (H)				(Pg 0046 00-1)