CAL 50 WRITTEN PE (JULY 2012)

1.	The top forward edges of the Breech Lock are beveled to?

a. Slow initial extraction	c. Fast extraction

b. Positive ejection		d. None of the above

2.	When the M2 is fully cocked, what component holds the firing pin extension in the rear position?

a. Accelerator stop		c. Cocking lever

b. Sear				d. Sear spring

3.	The purpose of the extractor assembly is to?

a. Extract a new round of ammunition from the belt

b. Extract the empty cartridge from the chamber

c. Actuate the feeding components in the cover

d. Prevent excessive gas pressure

4.	What is the corrective action for a damaged receiver?

a. (D)		b. (O)		c. (F)		d. (H)

5.	Replacement of the barrel locking spring is accomplished at?

a. (F)		b. (H)		c. (D)		d. (O)

6.	The first procedure when performing organizational maintenance is?

a. Fire the weapon		c. Set headspace

b. Clear the weapon		d. Ask the 1SG for a detail

7.	The technical manual used in this class was?

a. TM 9-1005-213-23&P		c. TM 9-1005-231-23&P

b. TM 9-1005-331-23&P		d. TM 9-1005-321-23&P

8.	Which of the following components are replaced at organizational level?

a. Rod, driving spring		c. Barrel extension

b. Extractor switch		d. Buffer assembly

9.	What level of maintenance is responsible to replace the extractor assembly?

a. (F)		b. (D)		c. (O)		d. (C)

10.	During the step of unlocking, what component moves the breech lock down?

a. Breech lock cam		c. Breech lock spring

b. Breech lock depressors	d. Breech lock push rod

11.	The belt feed pawl is not mission capable if? _______________________

12.	The purpose of the accelerator is to?

a. Help speed the bolt to the rear

b. Help lock the barrel to the barrel extension

c. Help speed the bolt forward

d. None of the above

13.	What component, if defective, could result in uncontrolled fire?

a. Firing pin		b. Sear		c. Breech lock		d. None of the above

14.	How many of the following parts could result in a failure to cock?

a. Broken cocking lever		c. Broken Sear

b. Defective cocking lever pin		d. All the above

15.	The probable cause for a ruptured cartridge is?

a. Defective safety		c. Improper headspace/timing

b. Defective sear		d. Defective firing pin

16.	What must be accomplished before inserting the headspace gage?

a. Remove the tool				c. Close the cover assembly

b. Cock and allow the bolt to go forward	d. Notify higher maintenance

17.	The sear spring is replaced by?

a. (F)		b. (H)		c. (O)		d. (D)

18.	The M2 Machine Gun is capable of feeding from the ___________ side?

a. Right	b. Left		c. Right or Left		d. None of the above

19.	Replacement of the M10 charging cable is accomplished by?

a. (D)		b. (H)		c. (O)		d. (F)

20.	Organizational PMCS is conducted?

a. Monthly		b. Quarterly		c. Weekly		d. Daily

21.	After the M2 Machine Gun has been reassembled, the armorer should?

a. Inform the 1SG			c. Clean the arms room

b. Conduct a daily inventory		d. Perform an operational check

22.	A defective sear could cause a failure to fire in what two steps in the cycle of function?

___________________	/	____________________

23.	What is the first step in troubleshooting a failure to eject? __________________

24.	Failure to extract would be caused by?

a. Pitted chamber			c. Burred or chipped threads

b. Burrs on bolt assembly		d. All of the above

25.	The M2 Machine Gun is ____________ operated?

a. Gas		b. Manually		c. Blowback w/ gas assist	d. Recoil