M249 WRITTEN PE (JULY 2012)

1.	The M249 is ____________ operated?

a. blowback		b. Recoil	c. Gas		d. Manually

2.	The armorer can replace which part?

a. Safety	b. Grip assembly	c. Sear spring		d. Trigger assembly

3.	The M249 Machine Gun is fired from the ________ bolt position?

a. Open	b. Closed	c. Centered	d. None of the above

4.	If the safety fails to slide properly after cleaning, what corrective action would you take?

a. Replace safety		c. Send to Direct Support

b. Send to Depot		d. None of the above

5.	What is the corrective action for a bent cartridge guide?

a. Replace	b. Hone	c. Direct Support	d. None of the above

6.	What is the corrective action for a completely plugged gas regulator (mono-block)?

a. (O)		b. (H)		c. (F)		d. (D)

7.	Replacement of a defective gas cylinder is by?

a. Operator	b. Direct Support	c. Armorer	d. Depot

8.	The sear is replaced by?

a. (F)		b. (O)		c. (C)		d. (H)

9.	The M249 Machine Gun

a. Is a semi-automatic weapon

b. Is belt or magazine fed

c. Fires from the closed bolt position

d. Is recoil operated

10.	The tripping lever is attached to the?

a. Sear		b. Trigger assembly		c. Safety	d. None of the above

11.	Which on of the following items is not replaceable at organizational level?

a. Frame trigger	b. Shoulder rest	c. Extractor	d. Recoil spring

12.	The armorer can replace the?

a. Bolt		b. Charging handle		c. Buffer		d. None of the above

13.	Replacement of the cover, magazine is done at?

a. (O)		b. (C)		c. (H)		d. (F)

14.	The first step in troubleshooting a failure to fire is?

a. Check for improperly installed firing pin

b. Check for loose gas collar

c. Check for broken firing pin

d. None of the above

15.	A worn or broken sear will result in a failure to?

a. Eject		b. Lock		c. Cock		d. Chamber

16.	If parts of the extractor group are defective from the bolt of the M249 Machine Gun, the armorer should?

a. Send to Direct Support

b. Replace with a new extractor kit

c. Replace the same pin back in the bolt

d. None of the above

17.	Which part of the gas system should never be lubricated?

a. Piston		b. Regulator		c. Gas tube		d. All of the above

18.	What is the NSN for the locking plate on bottom of the grip? ________________

19. 	PMCS for the M249 Machine Gun is scheduled on a ___________ basis?

a. Daily		b. Weekly		c. Quarterly		d. Monthly

20.	A broken driving spring could cause a failure to feed, what other cycle of function could it cause a failure in?

a. Cock		b. Fire		c. Unlock	d. Chamber

21.	What is the status symbol for a chipped ejector tip when the fault is listed on the DA Form 5988-E? _____________________

22.	What is the difference between the old and new style barrels? ___________________

23.	What is the NSN for the return rod and transfer assembly? ______________________

24.	What does the SMR code of XA mean? ___________________________

25.	What three parts of the trigger mechanism, if defective, could cause a failure to cock?

__________________, ____________________, and ___________________