M9 WRITTEN PE (JULY 2012)

1.	The M9 is ____________ operated?

a. Recoil	b.  Manually		c.  Gas		d. None of the above

2.	The M9 _________?

a. Is manually operated

b. Fires fully automatic with use of a selector switch

c. Is fed from 15 round magazine

d. Can be broken down into four (4) major groups

3.	Which of the following statements are true concerning the M9?

a. It is a fully automatic weapon

b. It is a semi-automatic weapon

c. All parts can be replaced at “O” maintenance

d. All of the above

4.	The NSN for the right hand pistol grip is? ____________________

5.	Replacement of the hammer is done by?

a. (H)		b. (O)		c. (F)		d. (D)

6.	What are the safety’s on the M9 pistol?

a. Decocking/safety lever, firing pin block

b. Grip safety, half cock and trigger lock

c. Half cock, grip safety and main safety lock

d. Main safety lock, grip safety and trigger lock

7.	When the hammer does not de-cock, the probable cause is?

a. Broken ejector	c. Dirt or obstruction in receiver

b. Broken firing pin	d. Defective trigger spring

8.	Maximum effective range of the M9 is?

a. 100 feet	c. 50 meters

b. 50 feet	d. 1800 meters

9.	Organizational maintenance is limited to?

a. Replacing pistol grips

b. Replacing grip washers

c. Reversing magazine release assembly

d. All the above

10.	When you see a TM number such as 9-1005-454-23&P, what does the last part represent (23&P)? _______________________________

11.	The M9 should be inspected _________________ by the Armorer?

a. Weekly	c. Monthly

b. Quarterly	d. Daily

12.	The corrective action for a damaged recoil spring is?

a. Organizational	c. General Support

b. Direct Support	d. Depot

13.	Corrective action for a damaged slide is?

a. Direct Support	c. General Support

b. Organizational	d. Depot

14.	After the last round has been fired, in what position will the slide be?

a. In the rear	c. Does not have a slide

b. Forward	d. Halfcocked

15.	Which of the following would be the most probable cause for a failure to feed on the M9?

a. Defective main spring

b. Worn disconnector

c. Defective magazine spring

d. Broken fixed ejector

16.	The trigger bar spring is replaced by?

a. (F)		c. (D)

b. (H)		d. (O)

17.	A dirty chamber could result in a failure to?

a. Eject		c. Extract

b. Cock		d. Fire

18.	A failure to unlock could be caused by?

a. Weak recoil spring

b. Locking block and Lug

c. Broken extractor

d. Obstruction in the barrel

19.	A broken trigger bar could result in a failure to?

a. Chamber	c. Fire

b. Lock		d. None of the above

20.	What are the differences between the M9 and the M9A1?

a. Sights

b. Trigger

c. Rail on the receiver

d. None of the above

21.	Quarterly PMCS inspection faults are recorded on?

a. DA Form 5990-E

b. DA Form 314

c. DA Form 5988-E

d. DA Form 2407

22.	A weak recoil spring could result in a failure to?

a. Cock		c. Fire

b. Chamber	d. Extract

23.	Which of the following would be the most probable cause for a failure to fire?

a. Weak recoil spring

b. Carbon accumulation in the firing pin strut

c. Broken firing pin

d. All the above

24.	Replacement of the firing pin is performed by _________ maintenance?

a. (H)		c. (C)

b. (F)		d. (O)

25.	Failure to fire in double action is caused by?

a. A broken trigger bar

b. A defective trigger bar spring

c. Decocking lever on safe

d. All the above