MK19 WRITTEN PE (JULY 2012)

1.	The MK19 is a _____________ operated weapon?

a. Recoil	b. Blowback	c. Manual	d. Gas

2.	The effective range of the MK19 is?

a. 1200 meters	b. 1000 meters	c. 1500 meters	d. 2200 meters

3.	The MK19 has ________ cycles of function?

a. 7		b. 8		c. 9		d. 6

4.	When the leading round is in line with the bolt face, describe the cycle of function?

a. Cocking	b. Charging	c. Feeding	d. Extracting

5.	The action taken for a runaway gun is?

a. Twist the ammo belt		c. Charging handles down

b. Expend all remaining ammo	d. Put on safe

6.	When removing the sear housing from the receiver, the safety should be on?

a. Fire		b. Safe		c. Doesn’t matter	d. Don’t know

7.	The MK19 is fired with the ____________?

a. Bolt to the rear	b. Bolt forward	c. Bolt half-cocked

8.	Which lubricant is prohibited for use on the MK19?

a. LSA		b. GMD	c. CLP		d. LSAT

9.	What component retracts the firing pin on the MK19?

a. Retractor assembly		c. Cocking assembly

b. Cocking lever		d. Sear

10.	How many inches of steel will the round penetrate?

a. 1		b. 3		c. 2		d. 4

11.	The maintenance level for the Ogive plunger is?

a. Operator	b. Organizational	c. Direct Support	d. General Support

12.	The cover hinge pins are replaced by?

a. (H)		b. (F)		c. (O)		d. (D)

13.	The vertical cam assembly is replaced by?

a. Direct Support	b. Organizational	c. Operator	d. Depot

14.	Replacement of the top cover is?

a. Operator	b. Direct Support	c. Organizational	d. General Support

15.	The internal lubrication of the sear assembly is performed by?

a. (F)		b. (D)		c. (O)		d. (H)

16.	The first step in troubleshooting sluggish firing is?

a. Bolt face dry		c. Recoil springs weak

b. Bolt rails burred		d. Carbon buildup in chamber

17.	Jammed bolt during charging could be caused by?

a. Vertical cam bent		c. Extractor spring defective

b. Cover hinge pin missing	d. Broken sear

18.	Probable cause of rounds not extracting would be?

a. Missing sear			c. Ejectors loose

b. Defective firing pin		d. Safety defective

19.	Feed slide adjustment with the adjustable secondary drive lever is performed by?

a. (H)		b. (O)		c. (C)		d. (F)

20.	What level of maintenance is responsible for lubricating the Ogive plunger?

a. (C)		b. (F)		c. (O)		d. (H)