M249 WRITTEN PE (DECEMBER 2017)
1.  The M249 is ______________ operated?


a.  Blowback

c.  Gas

b.  Recoil

d.  Manually

2.  The armorer can replace which part?

a.  Safety

     c.  Sear spring

b.  Grip assembly
     d.  None of these
3.  The M249 machine gun is fired from the ___________ bolt position?


a.  Open


c.  Centered


b.  Closed


d.  None of the above

4.  If the safety fails to slide properly after cleaning, what corrective action would you take?


a.  Replace safety

c.  Send to field maintenance

b.  Send to depot

d.  None of the above

5.  A damaged cartridge guide could cause a failure to? 


a.  Feed                    
c.  Fire
           b.  Extract                        d.  None of the above

6.  What is the corrective action for a plugged gas regulator (mono-block)?


a.  (O)

c.  (F)

b.  (H)

d.  (D)

7.  A failure to extract could be caused by?


a.  Ejector worn



c.  Loose tower

b.  Insufficient gas pressure

d.  Firing pin damaged
8.  A defective sear could cause a failure to? 

a.  Lock

c.  Cock

b.  Feed

d.  Unlock
9.  The M249 machine gun


a.  Is a semi-automatic weapon


b.  Is belt or magazine fed

c.  Fires from the close bolt position


d.  Is recoil operated

10.  The tripping lever is attached to the? 


a.  Sear

           c.  Safety


b.  Bolt


d.  None of the above

11.  A failure to charge could be caused by?


a.  Frame trigger

c.  Bent arm on the cocking handle

b.  Shoulder rest

d.  None of these
12.  The first check in a failure to chamber?

a.  Check for defective buffer       c.  Check for spent cartridge 




b.  Charging handle

          d.  None of these
13.  A failure to fire could be caused by?


a.  Firing pin tip damaged


c.  Bolt jammed

b.  Feed mechanism is sticking

d.  All of these
14.  The first step in troubleshooting a failure to fire is?


a.  Check for improperly installed firing pin


b.  Check for loose gas collar


c.  Check for broken firing pin

d.  None of the above

15.  A damaged sear will result in a failure to?


a.  Eject.

c.  Cock.


b.  Lock.

d.  Chamber.

16.  Extraction begins while the unlocking cycle in going on? 


a.  True

b.  False
17.  Which part of the gas system should never be lubricated?


a.  Piston


c.  Gas tube

b.  Regulator


d.  All of the above
18.  Where are the cleaning patches and CLP stored on the weapon? 


PISTOL GRIP
19.  PMCS for the M249 machine gun are scheduled on a __________ basis? 


a.  Daily


c.  Quarterly

b.  Weekly


d.  Monthly

20.  A broken driving spring could cause a failure to feed, what other cycle of function could it cause a failure in?


a.  Cock
.

c.  Unlock


b.  Fire


d.  Chamber
21.  What is the status symbol for a chipped ejector tip when the fault is listed on the DA Form 5988-E?


X
22.  What is the difference between the old and new style barrels?


MONO BLOCK  
23.  All barrels should be gauged and have a 100% technical inspection how often?

           ANNUALY
24.  What does the SMR code of XA mean? 

DO NOT REQUISITION
25.  What three parts of the trigger mechanism, if defective, could cause a failure to cock?

SEAR, SEAR SPRING AND TRIPING LEVER
1
2

