M16/M4 WRITTEN PE (DECEMBER 2017)
1. Which weapon has fire selector for Auto Fire?

a. M16A2
b. M4
c. M16A4
d. M4A1
2. A failure to cock in the M4A1 is most likely to occur because the

a. Firing pin has broken

b. The magazine catch is loose

c. The automatic sear is broken
d. The magazine spring is weak

3. Which statement is correct concerning the M4A1?

a. It is a gas piston operated weapons

b. It is a combination fixed blowback operated weapon

c. It is an air-cooled and recoil operated weapon

d. It is a gas operated, air cooled, lightweight, magazine-fed and semi-automatic or auto fire weapon

4. Which of the following parts are not interchangeable between weapons?

a. Bolt assemblies

b. Firing pin retaining pin

c. Handguards

d. The extractor

5. The most probable cause in a failure to extract would be?

a. Broken trigger

b. Defective action spring

c. Defective extractor spring assembly

d. Defective ejector spring

6. A bent gas tube in the receiver would cause a failure to?

a. Cock

c. Lock
b. Extract

d. Eject

7. The operator states his weapon does not chamber, therefore the armorer must troubleshoot the?

a. Parts in step of feeding steps 1 through 8

b. The parts of the firing mechanism

c. The action spring

d. The ejector spring

8. Which one of the following malfunctions could be caused by short recoil?

a. Failure to cock

b. Failure to extract

c. Failure to eject
d. All the above
9. Which part listed below if not installed will cause injury or death to the operator? (While firing the 1st round)

a. The automatic sear

b. The bolt carrier key

c. The firing pin

d. Bolt cam pin

10. In extreme arctic conditions the armorer would order which of the following lubricants for the M4A1?

a. GP oil

b. CLP 

c. LSA 

d. LAW

11. The first step in troubleshooting a failure to lock is? 

a. Short recoil
b. Check for broken action spring
c. Check for bent gas tube

d. Check for missing bolt cam pin

12. PMCS for the M4A1 is scheduled on a?

a. Quarterly
b. Monthly

c. Annually
d. Daily 

13. The third position of the SMR code determines?
a. How you get a part

b. Who can install, replace or use the item

c. Who determines disposition

d. Who can do the complete repair on the item 

14. Which part listed below could cause a loss of gas pressure?
a. Extractor pin
b. Trigger mechanism
c. Helical spring
d. Worn bolt rings
15. When parts of the upper receiver need replacement on the M4A1 the armorer using the SMR code may replace the?

a. Flash suppressor
b. Sear
c. None of these
d. Gas tube

16. What publication governs The Army Maintenance Management System (TAMMS)?
DA PAM 750-8
17. The armorer will request field maintenance to conduct gaging and 100% technical inspection how often?
a. Monthly
b. Semi-annually
c. Annually
d. Quarterly
18. Quarterly PMCS on the M4A1 best describe?

a. Finding and correcting small faults before they become major maintenance problems

b. Recording faults and informing the operator

c. Do PMCS so the 1SG can take action on cleaning

d. Primary maintenance concerning shortcomings

19. Disassembly of the lower receiver firing mechanism/trigger assembly is not authorized at organizational level because?

a. The special tools needed are too costly to have in each arms room

b. Repeated removal of the pins will enlarge the holes and result in malfunctions and loss of parts

c. Incorrect re-assembly would cause a failure to unlock

d. It is very hard for the unit armorer to learn

20. The hammer is cocked by the?

a. Bolt carrier

b. Locking lugs making contact with the barrel

c. By the expansion of the action spring

d. Pressure exerted by the bolt locking lugs

21. Broken or missing parts of the firing mechanism can cause a failure in what two steps of the cycle of function?

FIRING AND COCKING
22. What component provides the source of power to move the bolt carrier group forward after firing?

ACTION/BUFFER SPRING
23. A broken magazine catch could cause a failure in which?

a. Cock

b. Feed

c. Chamber

d. Fire

24. What is the difference between the NSN and the NIIN?

FIRST 4 DIGITS
25. What malfunction could occur if bolt rings are not staggered?

SHORT RECOIL
1
3

