MK19  WRITTEN PE

1.	How is the MK19 operated?

a. Manually

b. Blowback

c. Recoil

d. Gas

2.	 What is the maximum effective range in meters for point targets of the MK19?

a. 1000

b. 1050

c. 1500

d. 1550

3.	 How many cylcles of function does the MK19 have?

a. 6

b. 7

c. 8

d. 9

4.	When the leading round is in line with the bolt face, describe the cycle of function for the MK19?

a. Cocking

b. Charging

c. Feeding

d. Extracting

5.	What is the action taken for a MK19 runaway gun?

a. Twist ammo belt

b. Expend all remaining ammo

c. Charging handle down

d. Put on safe

6.	What position should the safety be on when removing the sear housing from the MK19 receiver?

a. Fire

b. Safe

c. Doesn't matter

7.	How is the MK19 fired?

a. With the bolt to the rear

b. With the bolt forward

c. With the bolt at half-cock

8.	Which lubricant is prohibited for use on the Mk19?

a. LSAT

b. LSA

c. GMD

d. CLP

9.	What component retracts the firing pin on the MK19?

a. Retractor assembly

b. Cocking lever

c. Cocking assembly

d. Sear

10.	How many inches of steel will the MK19 round penetrate?

a. 4

b. 3

c. 2

d. 1

11.	When must the MK19 Ogive plunger be lubricated?

a. Prior to firing

b. Semi-annually

c. Before cleaning

d. Never

12.	In what position must the MK19 cover be in before the hinge pins can be removed?

a. Closed

b. Open 90 degrees

c. Open 45 degrees

d. Doesn't matter

13.	Rounds will not feed on the MK19 could be caused by?

a. Contaminated bolt buffers

b. Feed slide mechanism out of adjustment

c. Burred feed tray

d. Either B or C

14.	The MK19 was originally developed by whom?

a. Browning

b. Air Force

c. Army

d. Navy

15.	The last MK19 round casing is?

a. Automatically ejected

b. Manually ejected

c. With a section of cleaning rod

d. Either B or C

16.	The 1st step in troubleshooting MK19 sluggish firing is?

a. Bolt face dry

b. Recoil springs weak

c. Carbon build-up in chamber

d. Bolt rails burred

17.	Jammed MK19 bolt during charging could be caused by?

a. Vertical cam bent

b. Cover hinge pin missing

c. Extractor spring defective

d. Broken sear

18.	Probable cause of the MK19 rounds not extracting could be?

a. Missing sear

b. Defective firing pin

c. Ejector loose

d. Defective safety

19.	Corrective action for the MK19 bolt timing out of adjustment is?

a. (H)

b. (O)

c. (C)

d. (F)

20.	What level of maintenance is responsible for lubricating the MK19 Ogive plunger?

a. (F)

b. (O)

c. (C)

d. (D)

21.	What MK19 component extends down the receiver's long axis and passes through the center of the bolt.

a. Sear assembly

b. Primary drive lever

c. Vertical Cam assembly

d. Backplate assembly

22.	What is the process of manually pulling the MK19 bolt to the rear?

a. Charging

b. Feeding

c. Firing

d. Extracting

23.	What malfunction can cause the bolt does not reach sear on the MK19?

a. Broken of missing spring washers

b. Burrs or aluminum buildup on vertical cam assembly

c. All of the above

d. None of the above

24.	Which assemblies of the MK19 should not be immersed in cleaning compound solvent?

a. Cam followers

b. Vertical cam assembly

c. Charging handles

d. All of the above

25.	What tool is the crew authorized to use on the MK19?

a. Tool, Ogive plunger assembly

b. Brush assembly, bore

c. Buffer tool

d. Wrench, spanner

26.	Of the following, which SMR code means items are subject to deterioration?

a. KD

b. PR

c. PC

d. AD

27.	What is the model number for the MK19 dummy cartridge?

a. M430/M430A1

b. M922/M922A1

c. M918 TP

d. M383

28.	What should you use to clean dirty ammunition?

a. Wire brush

b. CLP

c. Clean rag

d. All the above

29.	A two-man lift is required for the MK19 because it weighs how much?

a. 72.4 lbs

b. 75.3 lbs

c. 77.6 lbs

d. 78.8 lbs

30.	What assembly assists feeding of 40mm ammunition?

a. Sear assembly

b. Feed throat assembly

c. Backplate assembly

d. Alignment guide assembly