M17 WRITTEN PE

1.	During which cycle of function/operation on the M17 does pressing the trigger release the striker pin?

a. Unlocking

b. Firing

c. Extraction

d. Ejection

2.	During which cycle of function/operation on the M17 does the barrel unlocks from the slide and stops its rearward movement?

a. Firing

b. Unlocking

c. Feeding

d. Chambering

3.	During which cycle of function/operation on the M17 is the empty cartridge pulled from the chamber of the stationary barrel as the slide continues its rearward travel?

a. Extraction

b. Unlocking

c. Firing

d. Ejection

4.	What control/indicator on the M17 locks the magazine into the magazine well and releases the magazine

a. Takedown lever

b. Slide catch lever

c. Trigger

d. Magazine catch

5.	What control/indicator on the M17 blocks trigger bar from releasing trigger?

a. Slide catch lever

b. Loaded chamber indicator

c. Takedown lever

d. Manual safety lever

6.	What malfunction on the M17 could cause the cartridge not feeding?

a. Clean ammunition

b. Improperly assembled magazine

c. Wrong size grip module

d. Safety lever on "Safe"

7.	What malfunction could cause the cartridge not to chamber on the M17?

a. Clean ammunition

b. Obstruction in the chamber and bore

c. Safety lever in the "Safe" position

d. Sufficient lubrication

8.	What can cause the slide to not lock fully forward on the M17?

a. Safety lever in "Safe" position

b. Clean magazine

c. Improper size grip module

d. Dirty ammunition

9.	When troubleshooting the M17 pistol does not fire, what corrective action is taken when you find a broken trigger bar?

a. No action needed

b. Replace trigger bar

c. Replace magazine

d. Notify Field/ Maintainer maintenance

10.	What corrective action is needed when the cartridge does not extract because the chamber is dirty on the M17?

a. Notify Field/Maintainer maintenance

b. Clean chamber and barrel

c. Issue a different weapon

d. No corrective action is needed

11.	What malfunction on the M17 could cause a dead trigger?

a. Faulty ammunition

b. Takedown safety lever improperly positioned

c. Weak extractor spring

d. Operator headspace and timing

12.	On the M17, what does the front and rear sights contain?

a. Nothing

b. Whiteout

c. CLP

d. Tritium

13.	The M17 front sights illuminate what color?

a. Blue

b. Orange

c. Red

d. Green

14.	What is the first step when disassembling the M17 pistol?

a. Function check

b. Unload/clear weapon

c. Insert magazine

d. Place the safety lever on "Fire"

15.	The operator and/or the armorer can disassemble the M17 magazine?

a. True

b. False

16.	What is the first step when inspecting the slide and barrel assembly on the M17?

a. Inspect magazine

b. Inspect weapon for cleanliness

c. Check front and rear sights

d. Inspect recoil spring

17.	When inspecting the M17 recoil spring and guide assembly, what must you ensure on the recoil spring?

a. Magazine spring is free from damage

b. Free movement of slide catch lever

c. Not broken, bent or damaged

d. No cracks on receiver rails

18.	When inspecting the M17 receiver/grip module, you must ensure free movement of what component?

a. Magazine spring

b. Slide catch lever

c. Recoil spring guide

d. Barrel

19.	What lubricant is not authorized to use on the M17?

a. CLP

b. LAW

c. GMD

d. LSA

20.	On the M17, lightly lubricated means what?

a. Lubricant can be spread with a finger

b. Lubricant drips from weapon

c. Cannot see any lubricant film

d. Lubricant film barely visible to the eye

21.	How should the M17 barrel exterior be lubricated?

a. Lightly lubricated

b. Generously lubricated

c. Heavily lubricated

d. Lubricant is not authorized

22.	What should be lubricated on the M17 receiver/grip module assembly?

a. Recoil spring

b. Spring guide

c. Four receiver rails

d. Slide bushing

23.	What is the last step of the M17 assembly?

a. Install barrel

b. Install spring guide assembly

c. Perform safety/function check

d. Press down on slide catch lever

24.	When performing the safety/function check on the M17, what should happen when pressing the magazine catch?

a. Magazine should lock in place

b. Magazine must fall free under its own weight

c. The striker should release

d. Should here an audible click

25.	Which one of these can the unit armorer perform on the M17?

a. Grip module replacement

b. Disassemble the trigger mechanism

c. Reverse magazine catch

d. None of these

26.	When sizing the grip module on the M17 to the shooter, how should the shooters trigger finger lay on the trigger?

a. Shooter should alter grip to reach trigger

b. Entire finger should be across trigger

c. Finger should lay comfortably across trigger

d. It doesn't matter how finger is positioned

27.	How many grip assemblies are authorized for use on the M17?

a. 1

b. 2

c. 3

d. 4

28.	What is the NSN for the small arms swab for the M17?

a. 1005-00-716-2132

b. 1005-00-288-3565

c. 1005-00-556-4102

d. 1005-00-494-6602

29.	What is the term used to describe corrosion cracking and corrosion fatigue on the M17?

a. Stress

b. Pitting

c. Erosion

d. Fretting

30.	What control/indicator on the M17 provides a visual and physical indicator when a cartridge is loaded in the chamber?

a. Takedown lever

b. Slide catch lever

c. Magazine catch

d. Loaded chamber indicator