CAL 50: M2A1 WRITTEN PE

1. The M2A1 barrel extension includes a breech lock specifically fitted to maintain?

a. Slow initial extraction

b. Positive ejection

c, Fast extraction

d. M2A1 fixed headspace

2. When the M2A1 is fully cocked, what component holds the firing pin extension in the rear position?

a. Accelerator stop

b. Sear

c. Cocking lever

d. Sear spring

3. The purpose of the extractor assembly on the M2A1 is to?

a. Extract a new round of ammunition from the belt

b. Extract the empty cartridge from the chamber

c. Actuate the feeding components in the cover

d. Prevent excessive gas pressure

4. What is the corrective action for a damaged M2A1 receiver?

a. D

b. O

c. F

d. H

5. Adjustment of the headspace and timing on the M2A1 is performed by what maintenance level?

a. F

b. H

c. D

d. O

6. The first procedure when performing organizational maintenance on the M2A1 is?

a. Fire the weapon

b. Clear the weapon

c. Set headspace

d. Ask the 1SG for a detail

7. The Technical Manual for the M2A1 used in this class was?

a. TM9-1005-213-23&P

b. TM9-1005-331-23&P

c. TM9-1005-347-23&P

d. TM9-1005-321-23&P

8. How often is the headspace and timing checked on the M2A1?

a. Monthly

b. Prior to issuing/firing

c. Semi-annually

d. Not checked

9. The fault status symbol on the DA5988-E for a damaged M2A1 sear is?

a. /

b. X

c. O

d. -

10. During the step of unlocking on the M2A1, what component moves the breech lock down?

a. Breech lock cam

b. Breech lock depressors

c. Breech lock spring

d. Breech lock push rod

11. The belt feed pawl on the M2A1 is not mission capable if?

a. Binds

b. Fires

c. Slides

d. Moves

12. The purpose of the accelerator on the M2A1 is to?

a. Help speed the bolt to the rear

b. Help lock the barrel to the barrel extension

c. Help speed the bolt forward

d. None of the above

13. What component on the M2A1, if defective, could result in uncontrolled fire?

a. Firing pin

b. Sear

c. Breech lock

d. None of the above

14. How many of the following parts on the M2A1 could result in a failure to cock?

a. Broken cocking lever

b. Defective cocking lever pin

c. Defective firing pin extension

d. All of the above

15. The probable cause for a M2A1 ruptured cartridge is?

a. Defective safety

b. Defective sear

c. Improper headspace/timing

d. Defective firing pin

16. What must be accomplished before inserting the headspace gage on the M2A1?

a. Remove the barrel

b. Cock and allow the bolt to go forward

c. Close the cover assembly

d. Notify higher maintenance

17. The 50 Cal MG is NMC if the sear spring is?

a. Deformed

b. Incorrectly installed

c. Missing

d. All the above

18. The 50 Cal Machine Gun is capable of feeding from what side?

a. Right

b. Left

c. Right or Left

d. None of the above

19. PMCS on the M2A1 is conducted how often?

a. Monthly

b. Quarterly

c. Weekly

d. Semi-Annually

20. After the M2A1 Machine Gun has been reassembled, the armorer should?

a. Inform the 1SG

b. Conduct a daily inventory

c. Clean the arms room

d. Perform an operational/function check

21. A defective sear on the M2A1 could cause a failure to fire in what two steps in the cycle of function?

a. Firing and Feeding

b. Cocking and Ejecting

c. Firing and Cocking

d. Feeding and Cocking

22. On the M2A1, what is the first step in troubleshooting weapon will not eject?

a. Check for burrs in the T-slot

b. Check for defective ejector spring

c. Check for defective bolt ejector

d. Check for burred, broken/cracked, or bent firing pin

23. Failure to extract on the M2A1 would be caused by?

a. Pitted chamber

b. Burrs on rail of barrel extension assembly

c. Broken T-slot

d. All of the above

24. How is the M2A1 Machine Gun operated?

a. Gas

b.  Manually

c. Blowback w/gas assist

d. Recoil

25. What ensures that the Barrel Extension Assembly and Bolt on the M2A1 remain together as an assembly?

a. Serialized

b. Alphabetized

c. Initialized

d. Prioritized

26.  The M2A1, caliber .50, Flexible Barrel Machine Gun is?

a. a link-belt fed, recoil-operated, air-cooled, crew-served machine gun.

b. a magazine fed, recoil-operated, air-cooled, crew-served machine gun.

c. a magazine fed, gas-operated, air-cooled, crew-served machine gun.

d. a link-belt fed, recoil-operated, water-cooled, crew-served machine gun.

27.  The cyclic rate of this caliber .50 machine gun is how many rounds per minute.

a. 450-600

b. 500-650

c. 400-550

d. 550-700

28.  What is Step 3 when troubleshooting the malfunction BOLT WILL NOT LOCK on the M2A1?

a. Check for chipped, cracked, broken, or improperly assembled breech lock.

b. Check for burred, broken, or bent sear stop and pin.

c. Check for burred, cracked, chipped, or broken buffer accelerator.

d. Replace defective breech lock cam.

29.  What must be performed BEFORE beginning troubleshooting procedures on the M2A1?

a. Check headspace and timing.

b. Change barrels

c. Fire three rounds.

d. Mount weapon to M3 tripod

30.  According to the SMR code, what special tool is the Armorer authorized to have for the M2A1?

a. Gage, Wear Limit/Timig

b. Gage, Wear Check

c. Gage, Headspace

d. Gage, Breechbore