M240 SERIES MACHINE GUN WRITTEN PE

1.  The M240 MG is belt fed from what side?

a. Right side

b. Left side

c. Only magazine fed

d. Bottom

2.  The M240 Series Machine Gun employs what type of operation?

a. Recoil

b. Gas

c. Blowback

d. Manually

3.  The M240C Machine Gun is belt fed from what side?

a. Right side

b. Is capable of both right and left hand feed

c. It is manually fed

d. Left side

4.  IAW DA PAM 750-8, the completed Quarterly services are maintained for how long for the M240 series MG?

a. 180 days

b. 90 days

c. Until the next service is completed

d. 1 year

5.  The setting for normal rate of fire on the M240 gas regulator is in what position?

a. One

b. Two

c. Three

d. All of the above

6.  Preventive maintenance checks and services on the M240 series MG are performed how often?

a. Weekly

b. Monthly

c. Quarterly

d. Semi-Annually

7.  When installing barrel assembly into receiver on the M240 series MG, less than two or more than seven clicks indicates what?

a. Defective parts

b. That the barrel assembly is serviceable

c. That the receiver is serviceable

d. That the barrel release is serviceable

8.  Which part(s) should not be lubricated prior to firing the M240 series MG?

a. Trigger assembly

b. Bolt

c. Gas piston

d. Back plate

9.  The M240 gas regulator plug is designed with how many inlet settings to maintain the rate of fire?

a. One

b. Two

c. Three

d. Four

10.  Of the following items, which one is NMC on the M240 series MG?

a. Less than 1/3 exterior finish of the weapon exterior

b. Bipod leg damaged

c. Frayed sling

d. Missing barrel patches

11. IAW DA PAM 750-8, a completed/closed work request (DA 5990-E) on the M240 series MG is maintained for how long?

a. 1 year

b. 180 days

c. 2 years

d. 90 days

12.  How many current models of the M240 series machine gun are there?

a. 5

b. 6

c. 7

d. 8

13.  Sluggish operation on the M240 series MG could be caused by?

a. Loose bipod

b. Receiver dirty

c. Broken firing pin

d. Chipped extractor

14. An M240 series MG damaged gas plug could cause a failure to?

a. Extract

b. Cock

c. Chamber

d. Eject

15.  A runaway M240 series MG could be caused by?

a. Damaged recoil spring

b. Missing ejector

c. Worn sear

d. Damaged buffer

16.  The M240 Series Machine Gun fires from what bolt position?

a. Closed

b. Centered

c. Open

d. None of the above

17.  During what step in the cycle of operation for the M240 series MG does the ejector compress into the face of the bolt?

a. Locking

b. Ejecting

c. Feeding

d. Chambering

18.  Using the SMR code, which part(s) on the M240 series MG are replaceable by the armorer?

a. Buffer

b. Trigger spring

c. Extractor assembly

d. None of the above

19.  What is the source of power to move the M240 series MG bolt and operating rod assembly forward during chambering?

a. Gas pressure

b. Recoil of the bolt

c. Driving spring rod

d. All of the bolt

20.  What component holds the M240 series MG bolt and operating rod assembly to the rear in a cocked position?

a. Sear

b. Buffer

c. Gas pressure

d. Trigger pin

21.  Which of the following on the M240 series MG would cause a failure to feed?

a. Defective extractor

b. Defective barrel release

c. Damaged feed tray

d. Defective ejector spring

22.  What malfunction causes a failure to fire on the M240 series MG?

a. Driving spring rod assembly damaged

b. Loose rivets causing drag on bolt/operating rod assembly

c. Receiver dirty and/or lack of lubricant

d. All of the above

23.  Which of the following would NOT cause a failure to cock on the M40 series MG?

a. Broken parts in trigger

b. Worn firing pin

c. Broken sear spring

d. Broken or damaged operating rod

24.  Which of the following on the M240 series MG would cause a failure to eject?

a. Damaged ejector spring

b. Extractor frozen into the face of the bolt

c. Carbon accumulation on the firing pin aperture

d. None of the above

25.  Ruptured cartridges in the M240 series MG could be caused by?

a. Bent feed tray

b. Defective trigger

c. Improper headspace

d. None of the above

26.	What item on the M240 series MG serves as a support for all major components?

a. Receiver Assembly

b. Trigger

c. Buttstock

d. Cover Assembly

27.	What is the description of the buffer assembly for the M240 series MG?

a. Absorbs recoil for bolt and operating rod assembly at the end of recoil movement.

b. Controls the firing of the machine gun.

c. Serves as a guide for positioning cartridges to assist in chambering.

d. Provides energy for returning bolt and operating rod assembly to firing position.

28.	What component in the M240 series MG serves as a guide for positioning cartridges to assist in chambering?

a. Feed Tray

b. Sear spring

c. Receiver Assembly

d. Headspace

29.	What is the first step of functioning for the M240 series MG?

a. Chambering

b. Feeding

c. Cocking

d. Locking

30.	What cycle of function on the M240 series MG can only take place after locking of the bolt?

a. Unlocking

b. Feeding

c. Firing

d. Extracting