M249 WRITTEN PE

1.	How is the M249 operated?

a. Blowback

b. Recoil

c. Gas

d. Manually

2.	Using the SMR code, the armorer can replace which part on the M249?

a. Safety

b. Grip assembly

c. Sear spring

d. None of these

3. 	The M249 machine gun is fired from what bolt position?

a. Open

b. Closed

c. Centered

d. None of the above

4.  	If the safety on the M249 fails to slide properly after cleaning, what corrective action would you take?

a. Replace safety

b. Send to depot

c. Send to field maintenance

d. None of the above

5.  	A damaged feed tray could cause a failure to?

a. Feed

b. Extract

c. Fire

d. None of the above

6.  	What is the corrective action for a plugged gas regulator (mono-block) on the M249?

a. O

b. H

c. F

d. D

7.  	On the M249, a failure to extract could be caused by?

a. Ejector worn

b. Insufficient gas pressure

c. Loose tower

d. Broken extractor

8. 	A defective M249 sear could cause a failure to?

a. Lock

b. Feed

c. Cock

d. Unlock

9.  	The M249 machine gun

a. Is a semi-automatic weapon

b. Is belt or magazine fed

c. Fires from the close bolt position

d. Is recoil operated

10.  	The M249 tripping lever is attached to the?

a. Sear

b. Bolt

c. Safety

d. None of the above

11.  	On the M249, a failure to charge could be caused by?

a. Frame trigger

b. Shoulder rest

c. Bent arm on the cocking handle assembly

d. None of these

12.  	The first check in a failure to chamber the M249 is?

a. Check for defective buffer

b. Charging handle

c. Check for spent cartridge

d. None of these

13.  	A failure to fire the M249 could be caused by?

a. Firing pin tip damaged

b. Feed pawl assembly is sticking

c. Burred rear cartridge retaining pins

d. All of these

14.  	On the M249, the first step in troubleshooting a failure to fire is what?

a. Check for improperly installed firing pin

b. Check for loose gas collar

c. Check for broken firing pin

d. None of the above

15. 	 A damaged M249 sear will result in a failure to?

a. Eject

b. Lock

c. Cock

d. Chamber

16.  	The M249 extraction begins while unlocking is going on?

a. True

b. False

17.  	Which part of the M249 gas system should never be lubricated?

a. Gas Piston

b. Gas Regulator

c. Gas tube

d. All of the above

18.  	What component on the M249 houses the firing pin and roller assembly?

a. Bolt and piston assembly

b. Slide assembly

c. Feed tray assembly

d. Buttstock assembly

19.  	PMCS for the M249 machine gun are scheduled how often?

a. Semi-annual

b. Weekly

c. Quarterly

d. Monthly

20. 	 A broken driving spring could cause a failure to feed on the M249, what other cycle of function could it cause a failure in?

a. Cock

b. Fire

c. Unlock

d. Chamber

21.  	What is the status symbol for an M249 chipped ejector tip when the fault is listed on the DA Form 5988-E?

a. O

b. X

c. /

d. F

22.  	What is the difference between the old and new style M249 barrels?

a. Gas Regulator/Mono Block

b. Handle

c. Sights

d. Spring

23.  	All M249 barrels should be gauged and have a 100% technical inspection how often?

a. Semi-annually

b. Annually

c. Quarterly

d. Monthly

24.  	What does the SMR code of XA mean?

a. Do not requisition, order the next higher assembly

b. Order it using the CAGEC and part number

c. Identified by manufacturer's part number

d. Item is not stocked, local purchase

25.  	What three parts of the M249 trigger mechanism, if defective, could cause a failure to cock?

a. Sear, sear spring, trigger

b. Trigger, sear, safety

c. Sear, sear spring, tripping lever

d. Sear, safety, tripping lever

26.	The M249 cycle of functioning begins and ends when?

a. Sear, sear spring, tripping lever

b. At the open bolt position

c. Cocking

d. Feeding

27.	The M249 features fixed headspace which permits what?

a. Cocking and locking

b. Chambering and firing

c. Rapid barrel changes

d. Feeding and extracting

28.	What is the sustained rate of fire for the M249?

a. 1000 rpm

b. 85 rpm

c. 725 rpm

d. 100 rpm

29.	What two roles can the M249 operate in?

a. AR and LMG roles

b. AK and GML roles

c. OK and OMG roles

d. GP and AGR roles

30.	The EQUIPMENT NOT READY/AVAILABLE IF column indicates what?

a. Item to be checked or serviced

b. When to do the check or service

c. Deficiencies that must be corrected before MG can be operated

d. The M249 is returned to its containment area.