M16/M4 WRITTEN PE

1. Which weapon has fire selector for Auto Fire?

a. M16A2

b. M4

c. M16A4

d. M4A1

2. From the selection, a failure to cock in the M4A1 is most likely to occur because?

a. Firing pin has broken

b. The magazine catch is loose

c. The automatic sear is broken

d. The magazine spring is weak

3. Which statement is correct concerning the M4A1?

a. It is a gas piston operated weapons

b. It is a combination fixed blowback operated weapon

c. It is an air-cooled and recoil operated weapon

d. It is a gas operated, air cooled, lightweight, magazine-fed and semi-automatic or auto fire weapon

4. Which of the following parts are not interchangeable between weapons?

a. Bolt assemblies

b. Firing pin retaining pin

c. Handguards

d. The extractor

5. The most probable cause in a failure to extract on the M4A1 would be?

a. Broken trigger

b. Defective action spring

c. Defective extractor spring assembly

d. Defective ejector spring

6. A bent gas tube in the receiver of the M4A1 would cause a failure to?

a. Cock

b. Extract

c. Lock

d. Eject

7. The operator states his M4A1 weapon does not chamber, therefore the armorer must troubleshoot the?

a. Parts in step of feeding steps 1 through 8

b. The parts of the firing mechanism

c. The action spring

d. The ejector spring

8. Which one of the following malfunctions on the M4A1 could be caused by short recoil?

a. Failure to cock

b. Failure to extract

c. Failure to eject

d. All the above

9. Which part listed below if not installed on the M4A1 will cause injury or death to the operator while firing the 1st round?

a. The automatic sear

b. The bolt carrier key

c. The firing pin

d. Bolt cam pin

10. In cold arctic conditions, the armorer would order which of the following lubricants for the M4A1?

a. LSAT

b. CLP

c. LAW

d. LSA

11. On the M4A1, the first step in troubleshooting a failure to lock is?

a. Short recoil

b. Check for broken action spring

c. Check for bent gas tube

d. Check for missing bolt cam pin

12. PMCS for the M4A1 is scheduled how often?

a. Quarterly

b. Monthly

c. Annually

d. Weekly

13. The third position of the SMR code determines?

a. How you get a part

b. Who can install, replace or use the item

c. Who determines disposition

d. Who can do the complete repair on the item

14. Which part on the M4A1 listed below could cause a loss of gas pressure?

a. Extractor pin

b. Trigger mechanism

c. Helical spring

d. Worn bolt rings

15. When parts of the upper receiver need replacement on the M4A1, the armorer using the SMR code may replace the?

a. Flash suppressor

b. Sear

c. Sling

d. Gas tube

16. What publication governs The Army Maintenance Management System (TAMMS)?

a. AR 190-11

b. DA PAM 750-8

c. DA PAM 190-1

d. AR 190-13

17. The armorer will request field maintenance to conduct gaging and 100% technical inspection for the M4A1 how often?

a. Monthly

b. Semi-annually

c. Annually

d. Quarterly

18. Quarterly PMCS on the M4A1 best describe?

a. Any defects can be discovered and corrected before serious damage or failure occurs

b. Recording faults and informing the operator

c. Do PMCS so the 1SG can take action on cleaning

d. Primary maintenance concerning shortcomings

19. Each time a round is fired in an M4A1, the parts inside work together to accomplish their purpose in a given order is called what?

a. Feeding

b. Cycle of functioning

c. Chambering

d. Locking

20. The M4A1 hammer is cocked by what component?

a. Trigger

b. Sear

c. Bolt carrier

d. Recoil spring

21. What is the cycle of functioning when the M4A1 disconnector rotates to the rear and down, disengaging the hammer?

a. Burst Fire

b. Cocking

c. Ejecting

d. Extracting

22. What component provides the source of power to move the M4A1 bolt carrier group forward during chambering?

a. Trigger spring

b. Sear spring

c. Extractor spring

d. Action spring

23. A broken M4A1 magazine catch could cause a failure in which of the following?

a. Cocking

b. Feeding

c. Chambering

d. Firing

24. What is the difference between the NSN and the NIIN?

a. First four digits

b. Last four digits

c. No difference

d. Location of columns in the parts manual

25. What could occur if the M4A1 bolt rings are not staggered?

a. Loss of gas pressure

b. Will not feed

c. Will not fire

d. Cannot be placed on safe

26. What is the UOC for the M4A1?

a. AR8

b. AZ1

c. AS1

d. AY6

27. What is the cycle of functioning when the rearward motion of the M4A1 bolt carrier group withdraws the empty case from the chamber?

a. Unlocking

b. Extracting

c. Ejecting

d. Firing

28. During locking, the M4A1 bolt is maintained in its most forward position by what item?

a. Bolt cam pin

b. Bolt carrier key

c. Bolt carrier

d. Action spring

29. According to the SMR code, which part on the M4A1 can the armorer replace?

a. Bolt rings

b. Firing Pin

c. Magazine cartridge

d. None of these

30. During which cycle of functioning will the gas pressure cause the M4A1 bolt carrier group to move rearward?

a. Firing

b. Cocking

c. Chambering

d. Unlocking