UNIT ARMORER TEST

1.	Short recoil on the M4A1 could be caused by?

a. All of these

b. Damaged action spring

c. Missing bolt rings

d. Bent gas tube

2.	The MK19 has how many cycles of function/operation?

a. 7

b. 8

c. 6

d. 9

3.	If a round will not chamber on the M2A1, one of the probable causes is what?

a. Defective sear spring

b. Broken accelerator stop

c. Defective trigger bar

d. Defective extractor switch

4.	Services are scheduled how often, as a minimum, on the M2A1?

a. Annually

b. 60 days

c. Semi-annually

d. Quarterly

5.	The operator states that the safety fails to slide properly after cleaning and lubricating, the M249 should be?

a. Sent to field maintenance

b. Equipped with a new safety from stock

c. Nothing can be done

d. Disassemble and clean/lubricate the safety

6.	Failure to feed on the M240 could be caused by what?

a. Defective ejector

b. Damaged feed tray

c. Broken extractor

d. Defective firing pin

7.	When the M17 fails to fire, the probable cause is what?

a. A defective release lever spring

b. A broken slide

c. A broken trigger bar

d. An obstruction in the receiver

8.	What publication governs The Army Maintenance Management System (TAMMS)?

a. AR 750-35

b. DA PAM 710-1

c. DA PAM 750-8

d. AR 700-138

9.	The armorer will request Field Maintenance to Gage and conduct a 100% technical inspection how often?

a. Quarterly

b. Annually

c. Semi-Annually

d. Monthly

10.	The probable cause for a failure to fire on the M4A1 is what?

a. Bent gas tube

b. Damage firing pin

c. Loose extractor

d. Weak recoil spring

11.	The action taken for a runaway gun on the MK19 is what?

a. Twist the ammo belt

b. Charging handles down

c. Expend all remaining ammo

d. Put on safe

12.	When removing the sear housing from the MK19 receiver, the safety should be on what?

a. Fire

b. Safe

c. Doesn't matter

13.	What component retracts the firing pin on the MK19?

a. Retractor assembly

b. Sear

c. Cocking assembly

d. Cocking lever

14.	Probable cause of rounds not extracting on the MK19 is what?

a. Missing sear

b. Defective firing pin

c. Ejectors loose

d. Defective safety

15.	When the M2A1 fails to fire, one of the probable causes is what?

a. Defective belt feed pawl

b. Defective extractor switch

c. Defective sear

d. Defective bolt switch

16.	The M2A1 fails to cock, the probable cause could be what?

a. Defective trigger bar

b. Defective firing pin extension assembly

c. Defective barrel extension

d. Defective buffer

17.	The SMR code of "F" authorizes personnel to perform maintenance at what level?

a. Operator/Crew

b. Organization

c. Field/Maintainer

d. Depot

18.	The fault status symbol on the DA5988-E for a damaged M2A1 sear would be what?

a. /

b. X

c. O

d. F

19.	The M249 is what?

a. Automatic

b. Gas operated

c. Fired from the open bolt

d. All of these

20.	Quarterly preventive maintenance checks and services (PMCS) should be performed every how many days on the M249?

a. 30

b. 60

c. 90

d. 120

21.	When performing PMCS on the butt stock and shoulder assembly on the M249, those items should be inspected for what?

a. Cracks

b. Missing components

c. Shoulder piece locks in both up and down positions

d. All of these

22.	Sluggish operation of the M249 could be caused by what?

a. A bent cocking arm

b. Cracked buttstock

c. A bent feed tray

d. A dirty receiver assembly

23.	When the M249 fails to chamber a cartridge, the corrective action should be to check for what?

a. A dirty extractor

b. A broken ejector

c. A damaged helical compression spring

d. A broken sear

24.	When the M 249 fails to fire, the probable cause is what?

a. Carbon in gas inlets of regulator

b. Loose piston on operating rod

c. Damaged extractor

d. Damaged firing pin

25.	The first malfunction in troubleshooting a failure to fire on the M240 MG series is what?

a. Faulty ammunition

b. Carbon buildup in receiver

c. Improper lubrication

d. Damaged heat shield

26.	IAW DA PAM 750-8, how long are completed work orders (DA5990-E) maintained?

a. 1 year

b. 90 days

c. 60 days

d. 180 days

27.	What DA form is used to request field/maintainer maintenance?

a. DA 2404

b. DA 5988-E

c. DA 5990-E

d. DA 5513-E

28.	When is the status symbol "X" used?

a. TM stated NMC

b. Urgent MWO not applied

c. Safety

d. All of the above

29.	IAW DA PAM 750-8, how long are completed service packets (5988/5990) maintained?

a. 90 days

b. 180 days

c. 1 year

d. Until the next service is completed

30.	Which parts are not interchangeable between M4A1 weapons?

a. Handgaurds

b. Slings

c. Magazines

d. Bolts

31.	Which part if not installed on the M4A1 could cause catastrophic weapon failure when fired?

a. Bolt cam pin

b. Sear

c. Handgaurds

d. Charging handle

32.	Failure to feed on the M4A1 could be caused by what?

a. Defective magazine

b. Defective feed tray

c. Defective charging handle

d. Defective ejector

33.	PMCS on weapons is best described as?

a. Recording faults and informing the operator

b. Primary maintenance to repair weapons

c. Finding and correcting small faults before they become major maintenance problems

d. Cleaning weapons and turning them in to the arms room

34.	Which lubricant is prohibited on the MK19?

a. LAW

b. CLP

c. GMD

d. LSA

35.	How many inches of steel will the standard MK19 round penetrate?

a. 1

b. 2

c. 3

d. 4

36.	How is the MK19 automatic grenade launcher operated?

a. Gas

b. Recoil

c. Manually

d. Blowback

37.	The M240 MG is belt fed from which side?

a. Left side

b. Right side

c. Either left or right

38.	When installing the barrel on the M240B, less than two or more than seven clicks indicates what?

a. Barrel is serviceable

b. Receiver is serviceable

c. Defective parts

d. Barrel release is serviceable

39.	The weapons service paperwork/packets consists of what two forms?

a. DD214 and DD513

b. DD314 and DA 2404

c. Only the DA2407

d. DA5988-E and DA5990-E

40.	A malfunction that could cause a failure to fire on the M240 is what?

a. Driving spring is damaged or weak

b. Magazine is damaged

c. Charging handle is damaged

d. Short recoil

BARTON COMMUNITY COLLEGE

MILITARY SCHOOL

FORT RILEY, KANSAS

SMALL ARMS

FINAL EXAM 1

2 APRIL 2020

(ALL PREVIOUS EDITIONS ARE RESINDED)

2 APRIL 2020

Special Instructions:

This is an open book exam. You must use the reference publications shown below. You may use notes taken during class. DO NOT talk to other students at any time. Speak only to the Exam Proctor.

If you have any question, ask the Exam Proctor now.

Select the answer you believe to be correct. ONLY ONE correct answer is given for any question.

During the examination you will need:

A. Provided laptop with all regulation installed

B. An answer sheet

C. A pencil or pen